CARROLL ISD

ATHLETIC TURF GUIDELINES

AND

COACHES’ FACILITY MAINTENANCE EXPECTATIONS

Contents

PURPOSE	2

PREVENTIVE MAINTENANCE	3

LITTER REMOVAL	3

STAIN REMOVAL	4

MOSS, MOLD, ALGAE AND FUNGI REMOVAL	4

GRASS AND WEEDS REMOVAL	4

BRUSHING	6

COMPACTION RELIEVE	6

STATIC LOADS	6

VEHICULAR TRAFFIC	7

SPECIAL EVENTS	8

NON-PERMANENT FIELD MARKINGS	8

TREES AND OTHER VEGETATION IN ADJACENT AREAS	8

MINOR REPAIRS	10

# PURPOSE

- The goal of an artificial turf maintenance program is to ensure a consistent, safe, and attractive playing surface while also extending the life of the artificial turf system.

- Although artificial turf playing fields need little amount of attention in comparison with natural grass playing fields, they still require care and maintenance in order to avoid the deterioration of the playing surface. Playing fields that are not properly maintained will lose their original quality much sooner than fields that have been given proper attention on a regular basis.  Coaches must also take into consideration that Carroll ISD fields are in high demand from outside groups and will be utilized often by program enhancing youth teams and the community.

- Carroll ISD and the Southlake community has invested millions of dollars to ensure our athletic programs are afforded the best facilities possible.  The responsibility to properly care for these facilities starts with the Head Coach who will be held accountable for executing the maintenance and care program and ensuring the condition of the field.  It is recommended that Head Coaches take the lead in the daily/weekly routine of task while assigning assistant coaches to oversee certain duties as needed.  The student-athletes in the program should by assigned to daily duties as well. This will cut down on the time spent caring for the field, teach players responsibility, and increase the level of team pride in the facility.

- The following are keys to properly maintain an artificial playing field:

  - The artificial turf should be maintained free of all foreign material in order to avoid damage to the infill and fiber. Contamination of the infill may lead to hardening of the playing surface, decreased drainage, and growth of bacteria, grass and weeds.

  - The infill material should be maintained at the prescribed original level in order to guarantee a consistent shock absorbency and performance of the field while protecting the synthetic fibers.

  - The synthetic turf fibers shall be maintained in a vertical position. Fiber layover may lead to poor footing, decreased drainage, compaction and poor appearance.

  - The artificial turf should be maintained free of grass and weed growth.

  - No open flames, fireworks, welding, etc. shall be allowed on playing surface.

  - Static loads of more than 3psi should never be applied to the playing surface.

  - Transit of dynamic loads of more than 35psi should never be allowed on the playing surface.

  - No chemical agents or contaminated water should be applied to the playing surface.

  - Remove snow and ice with extreme care from the playing surface.

  - Report any minor or major damage immediately to Facility/Operations Administration.

  - Do not attempt repair without consulting with administration or the turf installation company.

  - Post the Turf Care Plan and duty assignments

  - Execute the Turf Care Plan consistently with great attention to detail and sense of pride.

- All heavy equipment and unnecessary vehicle traffic must be kept off the field. Also, sweeping should be avoided during the heat of the day.

- Before using any gas/oil powered equipment or vehicle on the turf, be sure to inspect all parts of the equipment to ensure there are no fuel/oil leaks that will damage the turf.

- Over time, the infill will settle to provide the best surface level for ideal comfort, playability and longevity of the system. This period of time varies, depending on weather conditions and use.  If there is a concern about any of these areas regarding infill level and/or specifications, contact Facility/Operations Administration.

# PREVENTIVE MAINTENANCE

- Help control access to the field.  Head coaches can check to see if there are outside groups scheduled for field use by checking the online “CISD Facility Scheduler” for usage schedules.  https://www.carrollisd.portal.rschooltoday.com/

- Prohibit food and beverage on the field.

- Prohibit chewing gum, seeds, and tobacco on the field.

- Prohibit open flames, fireworks, welding, etc. on the field.

- Keep all adjacent areas free of litter, debris, mud, dirt and spillage.

- Route field access traffic in such a way as to minimize mud and dirt tracking onto the field.

- Install trash and litter containers on the field and near the field in a sufficient number to avoid overflow of trash.

- Install water stations off the field to avoid contamination by spit.

- Set up drinks for athletics during practice breaks off the field.

- Prohibit all unnecessary vehicular traffic. Ban vehicles with non-pneumatic tires or with pneumatic tires with air pressure exceeding 35psi.

- Prohibit parking vehicles on the field.

- Prohibit changing or adding fluids to vehicles and equipment while on the turf surface as to prevent lubricating oil, grease, transmission fluid, etc. from dripping or spilling on the turf.

- Prohibit static loads on the field exceeding 3psi.

- Prohibit storage of materials on the field.

- Prohibit pets and animals from being on the field.

- Develop and implement a regular schedule of inspection and maintenance.

# LITTER REMOVAL

- A sweeper will be used to remove paper, sunflower seeds, leaves, etc. from the field.

- The brush will be equipped with synthetic fiber bristles of a minimum length of 2.5”.

- The brush will be set so that it barely touches the tips of the fibers of the turf. Never set the brush so low that it digs into the turf pile or infill material.

- Avoid using a motorized sweeper during the hottest time of the day.

- Remove ALL litter immediately after any event (including practice).

# STAIN REMOVAL

- Most stains on the turf fibers are not true stains, but rather residues of foreign matter.

- Prior to applying any chemical to the turf, contact Facility/Operations Administration.

- Water-based residues: Brush the residue with a fiber brush, sponge with neutral soap and water, rinse thoroughly.  DO NOT use a wire brush.

- Oil-based Residues: Contact Facility/Operations Administration.

- Animal Waste: sponge with mixture of white distilled vinegar in an equal amount of water, rinse thoroughly.

- Chewing Gum: spray with Freon and scrape to remove residue.

- Promptness is of the essence: it is easier to remove a fresh spill before it has time to dry and harden.

# MOSS, MOLD, ALGAE AND FUNGI REMOVAL

- Clean synthetic turf will not support the growth of moss, mold, algae and fungi.

- However, if organic matter (including food spills, leaves, etc.) is allowed to filter into the infill, moss, mold, algae or fungi may appear.

- Shade from trees and frequent irrigation can provide a cool, damp medium for moss, mold, algae or fungi growth.

- Appropriate non-oil-based chemical products may be used to remove moss, mold, algae or fungi growth. Rinse thoroughly.

- Even if living organisms are killed and removed, spores will remain. Therefore, successful treatment may require several applications.

- Promptness is of the essence: it is easier to remove moss, mold, algae and fungi growth before it becomes established.

# GRASS AND WEEDS REMOVAL

- Clean synthetic turf will not support the growth of grass and weeds.

- However, if organic matter (including food spills, leaves,etc) is allowed to filter into the infill, grass and weeds may appear.

- Shade from trees and frequent irrigation can provide a cool, damp medium for grass and weed growth.

- Remove vegetation growth by tearing the plants out manually.

- In extreme conditions, non-oil-based weed killer may be used for specific areas. Rinse thoroughly.

- Promptness is of the essence: it is easier to remove grass and weed growth before it becomes established.

Cleaning the turf is fairly easy. If there is surface soil, simply vacuum the turf like you would any carpet. This will lift away the soil and will not harm the fibers of the turf.

Removing Stains from Artificial Turf

There can also be minor spills that can occur on the turf. Our turf resists staining, but any spills should be cleaned properly.  Use a sponge with detergent suds and cold water within 24 hours of the spill. It can still be effective after 24 hours, but it is best to take care of the spill in a timely manner. Make sure not to use too much detergent when cleaning – use just enough to irrigate the solids out of the turf fibers. Too much can lead to a “ring” in your turf. To reduce the chance of a ring, limit detergent use and clean from the outside of the stain towards the center.

There are also more persistent stains and other problems that can affect the turf.  If a more persistent stain gets on your turf, sponge clean the area with a dry cleaning solvent. Make sure to follow the same process as detergent cleaning – making sure not to use too much and clean from the outside in. For nail polish, sponge with acetone.

A more persistent stain such as oil paint requires a little more work to clean out of the turf. Immediately blot the paint to remove as much as possible, then sponge the area with turpentine or paint remover. After using the paint remover, blot with detergent and cold water. Finally, re-sponge the area with cold water to ensure all the detergent and remover is out of the area. For an emulsified stain such as cosmetics, ink or show polish you start by sponging the area with detergent and cold water. Then apply a solvent to break up the stain before cleaning with soap and cold water, Finally, re-sponge the area with cold water to ensure all solvent and detergent have been cleared from the area.

Removing Foreign Substances from Artificial Turf

For foreign substances – such as chewing gum or tar – you will need to scrape the area to remove the gum or tar from the turf fibers. For chewing gum, first spray with Freon aerosol spray then scrape the gum free from the fibers. For tar removal, scrape the tar from the fibers first, then sponge with a dry cleaning solvent.

Taking these steps when cleaning the turf will ensure the best results. It cannot be guaranteed that stains will not happen after spills, but these cleaning instructions will help prevent them.  Cleaning all stains within 24 hours will get the best results.

# BRUSHING

- A turf groomer will be provided to maintain the distribution of the infill material in the field and to stand up the turf fibers.

- Maintaining a distribution of the infill material is essential to guarantee a consistent performance of the field and to properly protect the synthetic turf fibers.

- It is recommended to drag the field from side to side and to alternate dragging direction with each grooming.

- Follow manufacturer’s instructions for details. Adjust the aggressiveness of the groomer to a medium level. DO NOT DIG DEEP INTO FIBERS.

- If hollow infill spots are discovered, perform refill in several thin layers and brush infill material into the fibers with a fiber scrub brush or stiff broom.

- If high infill spots are discovered, vacuum the excess infill from the turf.

- Perform this operation twice monthly on football fields during heavy use times of the year and once a month in the off-season.  For baseball fields, during the season, the high traffic areas such as the pitching mound, bullpen mounds, batter’s box, catcher area, and base pads, should be checked and groomed DAILY.

# COMPACTION RELIEVE

- A tine rake shall be used to comb through the infill material in the field.

- Combing through the infill material is essential to relieve compaction and to assure a soft playing surface.

- Follow manufacturer’s instructions for details. Adjust the aggressiveness of the times to a medium level.

- It is not necessary to use the tine rake each time the turf is groomed. Perform this operation only twice a year.

# STATIC LOADS

- Static loads of more than 3 PSI should never be applied to the playing surface

- If needed, spread the static load on the playing surface through minimum ¾” thick plywood sheets, exterior grade. Place a polyethylene sheet under the plywood to avoid contamination of the turf.

- Remember that setup cranes, portable bleachers, concert stages and sound systems create high PSI levels.

- Remember that chairs and high heel shoes create high PSI levels.

- Never park vehicles or equipment on the playing surface.

# VEHICULAR TRAFFIC

- Transit or dynamic loads of more than 35 PSI should never be allowed on the playing surface.

- The loading of a pneumatic-tired vehicle is approximately equal to the air pressure in its tires.

- Vehicles with non-pneumatic tires shall never be allowed on the playing surface.

- No equipment other than designed maintenance equipment should be allowed on the playing surface.

- Maintenance equipment should be equipped with low-pressure pneumatic tires.

- Maintenance equipment should be operated avoiding excessive braking, turning, etc. that could damage the synthetic turf fibers and the base underneath. Move slowly and take wide turns.

- Hot engine exhaust should never be discharged into the field to prevent the possibility of burning or melting the synthetic turf fibers.

- Never leave a parked vehicle idling on the turf, in order to prevent the possibility of burning or melting the synthetic turf fibers due to exhaust or overheating.

- Never change or add fluids to equipment while on the turf surface as to prevent lubricating oil, grease, transmission fluid, etc. from dripping or spilling on the turf.

# SPECIAL EVENTS

- Remember that setup cranes, portable bleachers, concert stages and sound systems create high PSI levels.

- Remember that chairs and high heel shoes create high PSI levels.

- Large crowds shall never be allowed on the turf unless it is fully protected through minimum ¾” thick plywood sheets, exterior grade, mats or plastic tiles. Place a polyethylene sheet under the plywood, mats or plastic tiles to avoid contamination of the turf.

# NON_PERMANENT FIELD MARKINGS

- Absolutely NO markings painted on artificial turf fields without prior approval from Director of Facilities.

- Initially, polyolefin fibers will hold paint less adequately because they are coated with an additional inhibitor during the manufacturing process. As time progresses, the coating washes off the fibers and the fibers will hold paint better.

- Dilute, mix and install latex paint in accordance with the manufacturer’s instructions. Refer to the MSDS supplied by the paint manufacturer for special instruction, personal protective equipment, etc.

- Use a paint machine that allows for as low of a PSI level as possible.

- Apply paint on a clean and dry field only.

- Prior to painting, brush the area so that the fibers are vertical and the infill is loosened.

- Avoid applying thick layers of paint.

- Avoid driving paint into the infill.

- Allow a minimum cure time of 48 hours after striping before the field is put in service.

- Always avoid buildup of paint. Brush and clean the area with water and cleaning agents to remove excess paint existing on the field prior to applying new paint.

# TREES AND OTHER VEGETATION IN ADJACENT AREAS

- Tall trees can create a distracting shadow pattern on the field if the sun is behind them during play.

- Deciduous trees will drop leaves; evergreens will drop pinecones and needles. This organic matter might contaminate the infill.

- Shades from trees can provide a cool damp medium for moss, mold, algae, fungi, grass and weed growth.

- Tree roots may extend under the field. Where removing trees is not practical, root barriers may be installed.

- Whenever possible, keep trees a minimum of 10’ away from fields.

- Where synthetic turf abuts natural grass, grass and weeds may invade the edge of the synthetic turf.

- Where synthetic turf abuts natural grass, fertilizer, pesticides and other chemicals over spray may contaminate the turf.

- Wherever possible, install a minimum 10’ wide divider strip between synthetic turf and natural grass.

# MINOR REPAIRS

- Small loose spots on glued seams extending a few inches to two feet or rips and tears in the surface that do not exceed six inches should be regarded as minor problems unless allowed to grow. The Owner’s maintenance staff can generally repair these problems.

- Vacuum the infill material from the portion of the turf to be repaired.

- Dry-fit the fabric to be glued. It is recommended to have a 1/16” gap. Overlapping the seam may lead to lack of transfer of adhesive which might cause seams to open with use.

- Prop open the seam and insert approved seaming tape below the fabric to be glued. Allow for minimum six inches of seaming tape on each side of the seam.

- Make sure that the fabric to be glued and the seaming tape are perfectly dry and free of loose infill material, dirt, old adhesive or any other foreign matter.

- Mix and install approved adhesive on the seaming tape in accordance with the manufacturer’s instructions.

- Press the fabric into the adhesive bed uniformly.

- No fibers shall remain trapped by the adhesive.

- Weigh down the area and allow the adhesive to cure for at least 24 hours.

- Perform refill in several thin layers and brush infill material into the fibers with a fiber scrub brush.

Coaches Daily Checklist for Artificial Turf Fields

- Before you turn your back on that field, remember it still needs maintenance to keep it looking and performing its best.  It’s a huge investment that we want to make the most of.

- Remove all noticeable debris each day.  Have athletes walk the field for inspection before dismissal.

- Empty all trash can around the parameter of the field.

- Clean all storage areas each day.  Do not leave equipment out.  Athletes must pitch in to assist daily before dismissal.

- Inspect the field at least once a week (Monday) for tears or seam damage.

- Replace all displaced infill in high traffic areas.  Do this daily during season and at least twice a week during off-season (depending on use).

- Light Brush the field every other day to distribute infill and stand up fibers.

- Coaches must walk the field once each day to look for any stains, weed growth, or other potential problems.

- Coaches must inspect all safety netting in backstop cages, etc. each day to look for potential needed repairs and/or safety hazards.

- Remove all equipment from the playing surface each day.

- Clean and sweep the dugouts daily (athlete’s duty) to ensure no sunflower seeds or other debris gets into the turf fibers.

A checklist specific to each Facility and Sport will be provided to the head coach.  Record of routine maintenance and upkeep will be required and monitored.  The head coach will be responsible for the organization, plan, and follow through.  The head coach will provide a monthly maintenance checklist report to verify continuous and proper care is taking place.