Lead In Water Safety Program

Created March 2013 / Reviewed May 2024

District Facilities Department

Environmental Services Group

| Environmental Services Group |  |  | Applies to: ☐ ALL ☐ FP ☒ ESG ☐ NAC ☐ SEM ☐ FM ☐ Other | Number 026 |  |  | Rev. 02 |  | Page 1 of 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: Program Name |  |  |  |  |  |  |  |  |  |
| Type: ☐ Procedure ☒ SOP ☐ Contract ☐ Form ☐ Specification ☐ Design ☐ Accounting ☐ Planning ☐ Program ☐ Other |  |  |  |  | Department Owner: Environmental Services Group |  |  |  |  |
| REV |  | # DESCRIPTION OF CHANGE |  |  |  | DOC APPRV. |  | EFF. DATE |  |
| 1.0 |  | Written Program |  |  |  | TIS |  | 4/26/2016 |  |
| 2.0 |  | Review |  |  |  | CDN |  | 3/31/2016 |  |
| 3.0 |  | Review |  |  |  | MCS |  | 5/29/2018 |  |
| 4.0 |  | Review |  |  |  | MCS |  | 4/28/2020 |  |
| 5.0 |  | Review |  |  |  | ANM |  | 7/22/2022 |  |
| 6.0 |  | Review and Update |  |  |  | BTB |  | 5/15/2024 |  |
| 7.0 |  | Review |  |  |  | BTB |  | 10/20/2025 |  |
| DISTRICT PROGRAM FOR DEVELOPMENT AND IMPLIMENTATION OF PROCESSES TO BE USED IN SAINT PAUL PUBLIC SCHOOLS OPERATIONS |  |  |  |  |  |  |  |  |  |

# PURPOSE

Lead in Water

Saint Paul Public Schools (SPPS) is committed to providing a safe and healthful learning and work environment for all students and staff.  We recognize the importance of protecting our students and staff from potential lead exposure and will accomplish this via the following model plan.  The following Lead in Water Standard of Operation is provided as a guide to assist employers and employees of Saint Paul Public Schools in complying with Minnesota Statute 121A.335, and the Minnesota Department of Health (MDH) and the Minnesota Department of Education requirement for lead in drinking water for public schools, as well as to provide other helpful information.

Minnesota Statute 121A.335 requires public school buildings serving kindergarten through grade 12 to test for lead in water in potable water sources (consumable water) every 5 years.  The MDH and MDE have published Reducing Lead in Drinking Water: A Technical Guidance and Model Plan for Minnesota’s Public Schools, which presents a model plan that is available for schools to adopt.  Saint Paul Public Schools has used this to develop the following Lead in Water Safety Standard of Operation.

The District Environmental Services Group (ESG) will review the standard annually for particular requirements, which are applicable to the District and adjust this program accordingly.

The standard of operation is designed to be specific to the needs of users of Saint Paul Public Schools.

The standard of operation is intended to comply with the State of Minnesota’s requirements.

If there are conflicts or discrepancies between this standard of operation and the State of Minnesota’s requirements, the State of Minnesota shall take precedence.

# OBJECTIVE

- The objective of the Saint Paul School District Lead in Water Standard of Operation is to provide lead-safe drinking water sources throughout all of its facilities. Minnesota Statute 121A.335 requires lead concentration to be under 5 ppb in any tap that could be used as a drinking water source in a public school.  Environmental Services Group (ESG) will maintain a copy of this program.  A copy can be obtained from ESG.

# ASSIGNMENT OF RESPONSIBILITY

## Management

- Environmental Services Group (ESG)

- ESG is responsible for identifying potable water sources in all schools.

- ESG is responsible for accurately sampling and testing all schools with the aid of an accredited laboratory.

- ESG is responsible for immediately coordinating shutting off and remediating any drinking water source that tests above 5 ppb after results are received from the laboratory.

- ESG is responsible for maintaining sampling results and openly sharing results to the public.

# PROCEDURES

This program is modeled after the Minnesota Department of Health’s “Reducing Lead in Drinking Water: A Technical Guidance and Model Plan for Minnesota’s Public Schools”.  The program identifies drinking water sources, provides testing for these sources for lead and outlines follow-up procedures for fixtures testing high in lead content.  Saint Paul Public Schools will retest lead in water in each school at minimum every 5 years.

## Drinking Water Sources

SPPS recognizes that any fixture that can be used for consumption must be tested for lead in water.  These fixtures include:

- Drinking fountains and hydration stations

- Sinks in classrooms

- All kitchen area sources

- All nurse/heath area sources

- Water dispensers

- Office sinks

Other Water Fixtures

SPPS recognizes the following fixture types as water sources that are not used for the consumption of water. These fixtures are still sampled during the lead in water testing round for each building to better identify potential sources of lead in the plumbing system. Fixtures include:

- Restroom sinks

- Boiler room sinks

- Custodial slop sinks

Only the cold-water taps will be tested from all fixtures.  Additional sources may be added if they are identified as sources of consumption by facility occupants or district staff.

## Testing Methodology

Saint Paul Public Schools has outlined a testing protocol as follows:

- Sample collection will be performed in the morning, before other fixtures are operated, to capture a sample of the drinking water a person may consume from a fixture in regular operation.

- The water is the “first draw” of water from a fixture; “first draw” means that the samples are collected before the fixture is used or flushed for the day.  “First draw” represents a worst-case scenario.

- The sample collection size is 250 ml.

- The samples are sent to a MDH approved accredited lab for analysis.

- Do not flush the toilets or turn on the faucets for drinking water or washing hands until all samples are collected and tested.

- Samples are not kept for record after they have been tested.

- All drinking water sources shall be sampled, at minimum, every five years.

Sample Labeling

Each lab sheet shall be labeled with the following:

- Name of School and CAFM Number

- Date of Sampling

- Staff Who Conducted Sampling

- Sampled Fixtures with Associated Sample Numbers

  - Example Sample Number: #1020-001 – Dish Room Sprayer

## High Test Results and Remediation

Fixtures with results of 5 ppb or greater shall be turned off until replacement of the fixture is provided. Retesting shall occur to ensure fixtures tests below the action limit before they are deemed safe to drink from.

Resampling Procedure

Sampling after corrective actions follows the same procedure as stated above, but with an included fixture flush beforehand to recreate “regular use” of the fixture.

Taps must sit stagnant for a minimum of 6 hours but not more than 18 hours before sampling per MDH.

- Communication of Results

- SPPS maintains current lead in water testing information for each school on the SPPS Facilities webpage: https://www.spps.org/Page/3483.

- This web page provides the most current lead in water laboratory report for each school.  In addition, school principals will be notified of lead in water results for their respective school after the conclusion of all testing in that school.  A copy of SPPS Lead in Water Safety Program is also provided on the website.

# RECORDKEEPING REQUIREMENTS

## Management

Records of testing results will be maintained by ESG and can be accessed upon request.  Test results will be posted on Saint Paul Public Schools/Facilities website.

## Contact Information

Any questions, concerns, comments, or other information regarding the District Lead in Water SOP can be directed to:

- The Environmental Services Group

- Phone: 651.744.1800

- Email: ESG@spps.org

# RESOURCES & APPENDIX

## Reducing Lead in Drinking Water:

- Reducing Lead in Drinking Water: A Technical Guidance and Model Plan for Minnesota’s Public Schools.  Minnesota Department of Health.  Minnesota Department of Education.  April 2018.

## MDH Drinking Water in Schools for Educators and School Professionals:

- Minnesota Department of Health Drinking Water in Schools for Educators and School Professionals.  Minnesota Department of Health.  Accessed May 29, 2018.  http://www.health.state.mn.us/divs/eh/water/schools/.

## MDH Education and Communication Toolkit: Reducing Lead in Drinking Water

- Minnesota Department of Health Education and Communication Toolkit: Reducing Lead in Drinking Water.  Minnesota Department of Health.  Minnesota Department of Education.  Accessed May 29, 2018.  http://www.health.state.mn.us/divs/eh/water/schools/toolkit.pdf.

## EPA 3Ts for Reducing Lead in Drinking Water in Schools

- 3Ts for Reducing Lead in Drinking Water in Schools: Revised Technical Guidance.  Environmental Protection Agency.  October 2006.  Accessed May 29, 2018.  https://www.epa.gov/sites/production/files/2015-09/documents/toolkit_leadschools_guide_3ts_leadschools.pdf.