Pete Palmer, Director of Facilities • 760 Magnolia Avenue, Piedmont CA 94611• 510.594.2877

February 27th, 2026

City of Piedmont DigUSA requirments prior to start of work:

In Piedmont, California, you are legally required to contact Underground Service Alert (USA) North 811 before any excavation project, regardless of depth.

USAN 811 +2

Core Requirements

- Call 811 or Submit Online: You must notify the regional notification center by calling 811 or submitting an electronic ticket at least two working days (48 hours) before you plan to dig.

- Delineation: Before the utility locators arrive, you must mark the proposed excavation area in white using paint, flags, or stakes.

- Wait for Response: Do not begin digging until all utility operators have responded to your ticket. They will either mark their lines with color-coded paint/flags or notify you that they have no facilities in the area.

- Safe Digging (Tolerance Zone): If you are digging within 24 inches of a marked utility line, you must use hand tools only to carefully expose the line before using any power equipment.

- Ticket Validity: A USA ticket is valid for 28 calendar days. If your project exceeds this timeframe, you must renew the ticket.

DigAlert +8

Local Piedmont Resources

- Piedmont Building Permits: Many digging projects (like ADUs or on-grade improvements) also require a local permit from the City of Piedmont Planning & Building Department.

- Public Works Permits: For work involving sidewalks, curbs, or street use, contact the Piedmont Public Works Department at (510) 420-3050.

- Natural Gas: For specific gas line safety, refer to the Piedmont Natural Gas guidelines.

City of Piedmont CA (.gov) +5

Note: Failure to call 811 can result in fines up to $50,000 and liability for any repair costs if a utility line is damaged.

DigAlert +1