SECTION 08 31 00

ACCESS DOORS and FRAMES

PART 1	GENERAL

- RELATED DOCUMENTS

  - Drawings and general provisions of Contract, including General and Supplementary Conditions and Division 1 specification sections, apply to work in this section.

- SECTION INCLUDES

  - Access doors and frames at wall, ceiling, and floor locations.

- QUALITY ASSURANCE

  - Perform work in accordance with UL Design requirements.

- SUBMITTALS

  - Product Data: Provide data on unit construction, size, configuration, scheduled locations, and attachment method.

  - Manufacturer’s installation instructions:  Indicate any special installation criteria requiring an interface with adjacent components.

  - Contractor shall provide information so the Architect can record actual location of all access units on as-built drawings.

- REFERENCES

  - UL-Fire Resistance Directory

  - Warnock Hersey-Certification Listings

  - FBC - Florida Building Code

- REGULATORY REQUIREMENTS

  - Conform to applicable Code for UL and FM requirements for fire rated access doors.

PART 2	PRODUCTS

- ACCESS DOORS / HATCHES

  - Walls/Ceilings - Non-Fire Rated Door and Frame Unit: Formed steel, prime painted.

  - Walls/Ceilings - Fire Rated Door and Frame Unit: Formed steel, prime painted with rated label.

  - Floors, Non-Fire and Fire Rated Hatch and Frame Unit: Aluminum or Stainless Steel, fire-rating label as required.

- FABRICATION WALL / CEILING UNITS

  - Fabricate frames and flanges of 16-gauge steel.

  - Fabricate door panels of 14-gauge single thickness steel sheet or double sheet with integral non-combustible insulation filler.

  - Weld, fill, and grind joints to assure flush and square unit.

  - Hardware:

    - Hinge shall be 175° steel piano hinge with removable pin.

    - Lock shall have screwdriver slot for quarter turn cam-lock.

- FABRICATION FLOOR UNITS

  - Fabricate frames and flanges of ¼” aluminum.

  - Fabricate door panels of ¼” aluminum single thickness sheet or double sheet with integral non-combustible insulation filler.

  - Weld, fill, and grind joints to assure flush and square unit.

  - Hardware:

    - Hinge shall be 175° stainless steel piano hinge with removable pin.

    - Lock shall have screwdriver slot for quarter turn cam-lock.

- FINISHES

  - Steel protection shall be galvanized, hot dipped finish, prime coat units with alkyd primer.

  - Steel finish shall be two coats, baked enamel, and color as selected by the Architect.

  - Aluminum finish shall be factory mil finish.

PART 3	EXECUTION

- INSTALLATION

  - Install in accordance with manufacturer’s instructions.

  - Coordinate with work of other trades.

  - Verify sizes of openings prior to installation.

  - Install frames plumb and level in opening, and secure rigidly in place.

  - Position the door to provide convenient access to the concealed work-requiring access.

  - Provide minimum clear access opening of 24” X 24” with 180° swing on the door to access mechanical devices, see plans for required size.

- PROTECTION OF FINISHED WORK

  - Protect adjacent surfaces from damage by material installation.

  - Protect installation during construction activities.

  - Adjust unit for a smooth operation.

END OF SECTION