SECTION 08 33 13

COILING COUNTER DOORS

PART 1	GENERAL

- SECTION INCLUDES

  - Standard rolling counter doors; manual operated; factory pre-coated finish.

- REFERENCES

  - UL 325 – Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems

  - ASTM A123/A123M – Standard Specification for Zinc (Hot Dip Galvanized) Coating on Iron and Steel Products

- SYSTEM DESCRIPTION

  - Hand chain lift unit with overhead counter balance device.

  - Fire rated doors with fusible link activated with automatically governed closing speed.

- SUBMITTALS

  - Submit shop drawings and product data under provisions of Section 01 33 00.

  - Provide pertinent dimensioning, general construction, component connections and details, anchorage methods, hardware location and installation details.

  - Submit manufacturer's installation instructions under provisions of Section 01 33 00.

  - Submit sample of door for specified finish under provisions of Section 01 33 00.

- OPERATION AND MAINTENANCE DATA

  - Submit manufacturer's operation and maintenance data under provisions of Section 01 77 00.

PART 2	PRODUCTS

- MATERIALS

  - Curtain:

    - Slats: No. 1F interlocked flat-faced slats, 1½" high by ½" deep, 22-gauge AISI 300 series stainless steel with stainless steel angle bottom bar with lift handles and vinyl astragal.

    - Fabricate interlocking sections with high strength molded nylon end-locks riveted to ends of alternate slats.

    - Slat Finish: Stainless steel: No. 4 finish.

    - Bottom Bar Finish: Stainless steel: No. 4 finish.

  - Curtain Guides:

    - Stainless steel: 12-gauge formed shapes for integral frame and 14 gauge sill.

    - Finish: Stainless steel: No. 4 finish.

  - Counterbalance Shaft Assembly:

    - Barrel shall be steel pipe capable of supporting curtain load with maximum deflection of 0.03" per foot of width.

    - Spring Balance: Oil-tempered, heat-treated steel helical torsion spring assembly designed for proper balance of door to ensure that maximum effort to operate will not exceed 25 lbs.

      - Provide wheel for applying and adjusting spring torque.

  - Brackets: Fabricate from reinforced steel plate with bearings at rotating support points to support counterbalance shaft assembly and form end closures.

    - Finish: ASTM A123/A123M, Grade 85 zinc coating, hot dip galvanized after fabrication.

  - Hood: 24 gauge stainless steel with reinforced top and bottom edges and minimum ¼" steel intermediate support brackets as required to prevent excessive sag.

    - Finish: Stainless steel: No. 4 finish.

- ACCESSORIES

  - Locking:

    - Manual Crank Hoist: Pad lockable slide bolt on coil side of bottom bar of each jamb extending into slots in guides.

- OPERATION

  - Provide crank hoist operator, crank gearbox steel crank drive shaft, and geared reduction unit.

  - Fabricate gearbox to completely enclose operating mechanism and be oil-tight.

PART 3	EXECUTION

- EXAMINATION

  - Examine substrates upon which work will be installed and verify conditions are in accordance with approved shop drawings.

  - Coordinate with the Contractor to correct unsatisfactory substrates.

  - Commencement of work by installer is acceptance of substrate.

- INSTALLER

  - General: Install door and operating equipment with necessary hardware, anchors, inserts, hangers, and supports.

  - Follow manufacturer’s installation instructions.

- ADJUSTING

  - Following completion of installation, including related work by others, lubricate, test, and adjust doors for ease of operation, free from warp, twist, or distortion.

- CLEANING

  - Clean surfaces soiled by work as recommended by the manufacturer.

  - Remove surplus materials and debris from the site.

- DEMONSTRATION

  - Demonstrate proper operation and maintenance procedures to the Owner.

END OF SECTION