SECTION 08 33 23

OVERHEAD COILING DOORS

PART 1	GENERAL

- SECTION INCLUDES

  - Standard rolling service doors; manual operated; factory pre-coated finish.

- REFERENCES

  - UL 325 – Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems

  - ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process

  - FBC - Florida Building Code

- SYSTEM DESCRIPTION

  - Manual Push Up unit (Chain Hoist Operation), between jambs (surface) mounted.

  - Fire rated doors with fusible link activated with automatically governed closing speed.

- QUALITY ASSURANCE

  - ASCE 7 – Minimum Design Loads for Buildings and Other Structures

  - FBC Product Approval– Large Missile Impact Test if on exterior opening

- SUBMITTALS

  - Submit shop drawings and product data under provisions of Section 01 33 00.

  - Provide pertinent dimensioning, general construction, component connections and details, anchorage methods, hardware location and installation details.

  - Provide U.L. listing for fire rated units.

  - Submit manufacturer's installation instructions under provisions of Section 01 33 00.

  - Submit sample of door for specified finish under provisions of Section 01 33 00.

- OPERATION AND MAINTENANCE DATA

  - Submit manufacturer's operation and maintenance data under provisions of Section 01 77 00.

PART 2	PRODUCTS

- MATERIALS

  - Curtain:

    - Interlocking Roll-Formed Slats: Flat Profile Type F-265, 18-gauge galvanized steel.

    - Finish: Galvanized Steel in accordance with ASTM A653/A653M and receive rust-inhibitive, roll coating process, 0.2 mils thick baked on prime paint, and 0.6 mils thick baked-on polyester (powder coated) top coat.

  - Curtain Guides:

    - Roll-formed galvanized steel shapes attached to continuous galvanized steel wall angle.

  - Counterbalance Shaft Assembly:

    - Helical torsion spring type designed for standard 50,000-cycle life design.

    - Counterbalance shall be housed in a steel tube, supporting the curtain with deflection limited to 0.03 per foot of span.

    - Counterbalance shall be adjustable by means of an adjusting tension wheel.

  - Brackets: Fabricate from galvanized steel to support counterbalance, curtain, and hood.

  - Hood: 18-gauge galvanized steel with reinforced top and bottom edges.

    - Provide min ¼-inch steel intermediate support brackets as required to prevent excessive sag.

  - Bottom Bar: Two galvanized steel angles.

- ACCESSORIES

  - Locking: Pad lockable slide bolt on coil side of bottom bar of each jamb.

- OPERATION

  - Provide manual push up (Chain Hoist) operation.

  - Mount chain hoist in conformance with current ADA Guidelines.

# PART 3	EXECUTION

- EXAMINATION

  - Examine substrates upon which work will be installed and verify conditions are in accordance with approved shop drawings.

  - Coordinate with the Contractor to correct unsatisfactory substrates.

  - Commencement of work by installer is acceptance of substrate.

- INSTALLER

  - Install door and operating equipment with necessary hardware, anchors, inserts, hangers, and supports.

  - Follow manufacturer’s installation instructions.

- ADJUSTING

  - Following completion of installation, including related work by others, lubricate, test, and adjust doors for ease of operation, free from warp, twist, or distortion.

- CLEANING

  - Clean surfaces soiled by work as recommended by the manufacturer.

  - Remove surplus materials and debris from the site.

- DEMONSTRATION

  - Demonstrate proper operation and maintenance procedures to the Owner.

END OF SECTION