SECTION 08 33 26

OVERHEAD COILING GRILLES

PART 1	GENERAL

- SECTION INCLUDES

  - Standard overhead coiling grilles; manual operated; stainless steel finish.

- REFERENCES

  - UL 325 – Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems

  - ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process

  - FBC – Florida Building Code

- SYSTEM DESCRIPTION

  - Manual Push Up unit (Chain Hoist Operation), between jambs (surface) mounted.

- SUBMITTALS

  - Submit shop drawings and product data under provisions of Section 01 33 00.

  - Provide pertinent dimensioning, general construction, component connections and details, anchorage methods, hardware location and installation details.

  - Submit manufacturer's installation instructions under provisions of Section 01 33 00.

  - Submit sample of door for specified finish under provisions of Section 01 33 00.

- OPERATION AND MAINTENANCE DATA

  - Submit manufacturer's operation and maintenance data under provisions of Section 01 77 00.

PART 2	PRODUCTS

- SECURITY GRILLES (SEE DOOR SCHEDULE FOR LOCATIONS)

  - Curtain:

    - Stainless steel grille constructed of 7/16" diameter horizontal members spaced at 1½" centers with ½" diameter tube spacers for steel carrier rods spaced at 3 ¼" centers.

    - Vertical connect member with a hinged type design of ⅝" x ⅛" flat bar and links spaced at 6" centers.

    - Fit ends of horizontal members with nylon runners for quiet operation; provide bottom bar of back-to-back angles with tubular resilient cushion.

  - Guides:  Stainless steel angles of profile to retain curtain in place.

  - Crosshead Counterbalance Shaft:

    - Provide steel pipe with closed ends of sufficient diameter to ensure minimum deflection of .03" per foot.

    - Balance with adjustable spring tension provided by helical steel springs to produce sufficient torque assuring smooth, correct operation of grille from any position.

  - Hood:

    - 24-gage galvanized steel reinforced with stiffening seams at top and bottom edges for enclosing coiled curtain.

    - Provide intermediate support for grilles over 14'-0".

  - Brackets:  ¼" thick steel plate designed to support guide extensions and form end closure support for hood.

  - Hardware:  Locks:  Slide bolts, interior side, bottom in accordance with Section 08 71 00.

- FINISH

  - Stainless steel, Type 304, with No. 4 satin finish

- OPERATION

  - Provide manual push up (Chain Hoist) operation.

  - Mount chain hoist in conformance with current ADA Guidelines.

PART 3	EXECUTION

- EXAMINATION

  - Examine substrates upon which work will be installed and verify conditions are in accordance with approved shop drawings.

  - Coordinate with the Contractor to correct unsatisfactory substrates.

  - Commencement of work by installer is acceptance of substrate.

- INSTALLER

  - General: Install door and operating equipment with necessary hardware, anchors, inserts, hangers, and supports.

  - Follow manufacturer’s installation instructions.

- ADJUSTING

  - Following completion of installation, including related work by others, lubricate, test, and adjust doors for ease of operation, free from warp, twist, or distortion.

- CLEANING

  - Clean surfaces soiled by work as recommended by the manufacturer.

  - Remove surplus materials and debris from the site.

- DEMONSTRATION

  - Demonstrate proper operation and maintenance procedures to the Owner.

END OF SECTION