SECTION 08 71 00

DOOR HARDWARE

PART 1	GENERAL

- SECTION INCLUDES:

  - Hardware for wood and hollow steel doors

  - Lock Cylinders for gates, folding partitions, wire cages, and doors

  - Thresholds

  - Gaskets

- REFERENCES

  - NFPA 80 – Standard for Fire Doors and Other Opening Protectives

  - AWI - Architectural Woodwork Institute

  - BHMA - Builders' Hardware Manufacturers Association

  - DHI - Door and Hardware Institute

  - Florida Building Code: Accessibility NAAMM - National Association of Architectural Metal Manufacturers

  - FFPC – Florida Fire Prevention Code

  - SDI - Steel Door Institute

  - FBC - Florida Building Code

  - FFPC _ Florida Fire Prevention Code

- COORDINATION

  - Coordinate work of this Section with other directly affected Sections involving manufacturer of any internal reinforcement for door hardware.

- QUALITY ASSURANCE

  - Manufacturers:  Companies specializing in manufacturing door hardware with minimum 5-years of experience.

  - Hardware Supplier:  Company specializing in supplying institutional door hardware with minimum 5-years documented experience.

  - Hardware Supplier Personnel:  Employ an Architectural Hardware Consultant (AHC) to assist in the work of this Section and be readily available for consultation.

- REGULATORY REQUIREMENTS

  - Conform to the FBC for requirements applicable to both fire rated doors/frames and accessibility for the physically disabled.

  - Conform to the applicable sections of FFPC.

- CERTIFICATIONS

  - Architectural Hardware Consultant shall inspect complete installation and certify that hardware and installation is in accordance with manufacturer's instructions and this specification.

  - Provide two copies of certifications to the Architect.

- SUBMITTALS

  - Submit schedule and product data under provisions of Section 01 33 00.

  - Indicate locations and mounting heights of each type of hardware.

  - Provide product data on specified hardware.

  - Submit samples under provisions of Section 01 33 00.

  - Submit samples of hinge, latch set, exit device, door closer, threshold, illustrating style, color, and finish.

  - Samples:  Incorporate the samples within the Work.

  - Submit manufacturer's parts lists, templates, and installation instructions under provisions of Section 01 33 00.

  - Submit manufacturer's certificate under provisions of Section 01 40 00 that fire rated hardware meets or exceeds specified requirements.

- OPERATION AND MAINTENANCE DATA

  - Submit operation and maintenance data under provisions of Section 01 77 00.

  - Include data on operating hardware, and inspection procedures related to preventative maintenance.

- DELIVERY, STORAGE, AND HANDLING

  - Deliver products to site under provisions of Section 01 60 00.

  - Store and protect products under provisions of Section 01 60 00.

  - Package hardware items individually, label and identify package with door opening code and hardware group to match hardware schedule.

  - Deliver all final keys and construction key voiding devices to Owner's Lock Department by security shipment direct from hardware supplier.

    - Delivery Address:

The School District of Palm Beach County

Department of Facilities Services

Quality Assurance Section

West Palm Beach, Florida 33406-4108

  - Deliver two copies of factory key biting schedule to the Owner's Lock Department in conjunction with delivery of final keys.

  - Protect hardware from theft by cataloging and storing in secure area.

- WARRANTY

  - Provide a minimum of a 5-year warranty period under provisions of Section 01 77 00.

  - Warranty: Include coverage of door closers, locksets, latch sets, exit devices hinges and all items listed in the hardware schedule

- MAINTENANCE MATERIALS

  - Provide special wrenches and tools applicable to different or special hardware component.

  - Provide maintenance tools and accessories supplied by hardware component manufacturer.

PART 2	PRODUCTS

- ACCEPTABLE MANUFACTURERS

  - Hinges:  Ives, Hager, Stanley

  - Reinforcing Pivots:  Hager

  - Cylinder Locks:  Schlage

  - Latch Sets:  Schlage

  - Cabinet Locks:  Schlage, Olympus with Schlage IC Core

  - Rim / Mortise Cylinders:  Schlage IC Core

  - Mortise Locks:  Schlage

  - Exit Devices:  Von Duprin, Falcon – using Schlage IC Core

  - Door Closers: LCN

  - Removable Mullions: Von Duprin, Falcon; using Schlage IC Core

  - Manual/Automatic Bolts: Ives, DCI

  - Coordinators:  Ives, DCI

  - Push/Pulls: Ives, Quality, Rockwood

  - Protection Plates: Ives, Quality, Rockwood

  - Gaskets / Thresholds: Zero, National Guard

  - Power Supplies: Von Duprin

  - Power Transfer: Von Duprin

- HINGES

  - Size: Provide 4½" wide x 4½" high, 5BB1 at exterior and 5BB1 at interior locations.

  - No less than three hinges on any door and add a reinforcing pivot on all doors over 3' 0" wide.

  - Ball Bearing hinges on doors with door closers and provide Non-Removable Pin type on all exterior out-swing doors.

- LOCKSETS

  - Supply in the following functions:

LOCATION	FUNCTION

    - Passage	ND10S

    - Privacy	ND40S(44S in Grades K-2)

    - Offices / Classrooms (security lock)	ND95PD x RHO x 626

    - Storeroom/Mechanical Room	ND96PD x RHO x 626

    - Deadlocks (student areas)	B663P

    - Deadlocks (non-student areas)	B660P

    - Staff Restrooms	L9496 w/ Occupied Indicator, L9496-P-06A-626

  - Electronic Lever Lock: Schlage L9080PEU x 06 x 630 or ND96PDEU X RHO X 626

- DOOR CLOSERS

  - All door closers shall be a LCN 4040XP/4041XP series.

  - Furnish Parallel arms on all doors scheduled for closers except where they are mounted showing in corridors.

  - Supply nylon spacer for fifth mounting screw on parallel arm installations where required due to frame configuration.

  - Supply with EDA arm on installations in main corridor entrances, gymnasiums, locker rooms, and restrooms.

- EXIT DEVICES AND POWER SUPPLY

  - Supply the following as applicable:

TYPE DOOR	DEVICE

    - Non-fire rated	Von Duprin 98/99 with Trim as specified

    - Fire-rated	Von Duprin 98/99 with Trim as specified

    - Non-fire rated* (pairs)	Von Duprin 98/99 with Trim and Mullion as specified

    - Fire-rated*(pairs)	Von Duprin 98/99 with Trim and Mullion as specified

    - Fire-rated (Electronic)	Von Duprin 98/99 QEL-98L-F x E996L x 06 or E996 Trim as specified

    - Non-fire-rated (Electronic)	Von Duprin 98/99 QEL or E996 Trim as specified

    - Power Supply	Von Duprin PS900 Series

    - Electrical Power Transfer	Von Duprin EPT-10 X SP 28EPT10 x SP28

* Note – Interior doors (pairs) shall be equipped with removable mullions with key removable operation (with IC cylinders).

* Note- Exterior doors (pairs) shall be non-keyed removable per Miami-Dade County NOA requirements.

- DOOR TRIM

  - All push plates, pull plates and kick plates manufactured of .050 stainless steel.

  - Push plates and pull plates 4" wide x 16" high.

  - Kick plates shall be 10" high x 2" less than door width.

2.7 DOOR STOPS

A. Door stops to be of the following types;

1. Manufacturers; Ives, Rockwood

B. Overhead Stop and Holders;

1. Manufacturers; GLYNN Johnson, Rixson

2.8 AUTOMATIC FLUSH BOLTS, SURFACE BOLTS AND COORDINATORS

A. Supply door bolts of the following type;

1. Manufacturers; H.B. Ives, DCI

a. H.B Ives

b. DCI

2.9 THRESHOLDS AND SEALS

A. Supply threshold and seals of the following types for exterior doors (per current Miami-Dade County NOA requirements)

1. Manufacturers; NGP Products, Zero International

a. NGP Products;

b. Zero International

B. Supply thresholds and seals of the following types for interior doors (as scheduled in hardware sets);

1. Manufacturers; NGP, Zero International

2.10 OVERHEAD RAIN DRIP

A. Provide overhead rain drip of the following types at all exterior door frame locations or as scheduled in these specifications (per Miami-Dade County NOA requirements);

1. Manufacturers; Zero International, NGP Products

2.11 FASTENINGS

A. All screws of matching finish to their product and to the manufacturer’s standards for that item and its intended use.

2.12 KEYING

- Pre-Order Meeting:  Hardware Supplier shall meet with a representative of the Owner's Lock Department and Department Head to establish a keying schedule before placing any orders.

- Locks:

    - Provide keying as established in pre-order meeting with Hardware Supplier.

    - All locks to be Construction Master Keyed using the split key method.

- All locks and cylinders factory keyed to Palm Beach County School District Schlage EVEREST D245 restricted keyway.

- Hardware Supplier and a representative of the Owner's Lock Department will meet and establish final count of locks and cylinders.

    - The Owner's Lock Department will transmit a letter to the Schlage Lock Company releasing locks and cylinders in the amount established in the final count to the Hardware Supplier.

- Supply two keys for each lock with a maximum of four keys of keyed alike sets.

- Supply keys in the following quantities:

    - Five Master Keys

    - Five Grand Master Keys

    - Five Great Grand Master Keys

    - Two Control Keys (for interchangeable cores)

    - Twelve Construction Master Keys

- Key Delivery: Refer to Section 08 71 00 1.09, D.

- Visual Key control: Use visual key control – stamp keys only.  Stamp all keys “Do Not Duplicate”.

2.13 KEY CABINETS

- Key Cabinet: Sheet steel construction, piano hinged door with pin tumbler lock of brass construction.

- Cabinet Size: Provide a size for known project keys plus 10%.

- Horizontal metal strips for key hook labeling with clear plastic strip cover over labels.

- Finish shall be baked enamel finish, gray color.

  - KNOX BOX

- Provide the required Knox box as manufactured by Knox Company to meet the local Fire Department criteria.

- Coordinate ordering and locating with the Fire Department having jurisdiction.

- Provide a Knox box as manufactured by Knox Company to meet the School District’s Police Department criteria.

    - Coordinate ordering and locating with the School District’s Police Department

- Provide two Knox padlocks to meet the local Fire Department criteria.

    - Coordinate ordering and locating with the Fire Department having jurisdiction.

  - FINISHES

- Exterior Hinges: US32D

- Interior Hinges: US26D

- Exterior Locks: US32D

- Interior Locks: US26D

- Coordinators: US26D

- Push, Pull & Kick Plates: US32D

- Door Closers: AL

- Exit Devices: US32D/US26D

- Door Stops & Holders: US32D

- Thresholds &Weather Strips: AL

  - CLOSERS, MAGNETIC HOLD-OPEN SYSTEMS:

- Furnish closer/electromagnet complete with required accessories necessary for a complete working system.  Provide a two-year warranty.

    - Manufacturers; LCN, Rixson

      - LCN, SEM 7800

      - Rixon, 998

PART 3	EXECUTION

- INSPECTION

  - Verify that doors and frames are ready to receive work and dimensions are as indicated on shop drawings.

  - Beginning of installation means acceptance of existing conditions.

  - Do not install any hardware until after the hardware manufacturer’s representative pre-installation class to ensure proper installation of the specified products.

- INSTALLATION

  - Install hardware in accordance with manufacturer's instructions and requirements of DHI.

  - Use the templates provided by hardware item manufacturer.

  - Mounting heights for hardware from finished floor to center line of hardware item:

    - Locksets:	38 inches

    - Push/Pulls:	42 inches

    - Dead Locks:	48 inches

    - Exit Devices:	38 inches

  - Conform to Florida Accessibility Code for Building Construction for positioning requirements for the disabled.

  - Set all thresholds in a full bed of butyl rubber.

  - The hardware manufacturers will provide a post installation review to ensure proper installation of the specified products.

- ADJUST AND CLEAN

  - Adjust and check the operation of each item of hardware and each door, to ensure the proper function of every item.

    - Adjust or replace all items that to operate freely and smoothly.

  - Made final adjustments after all ventilating systems are in operation.

  - Clean all hardware and adjacent surfaces after hardware installation.

  - Instruct Owner's personnel in adjustment and maintenance of hardware and hardware finishes.

END OF SECTION