SECTION 09 30 13

CERAMIC and QUARRY TILE

PART 1	GENERAL

- RELATED DOCUMENTS

  - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 specification sections, apply to work in this section.

- SECTION INCLUDES

  - This Contractor shall furnish and install all tile and accessories shown on the plans or specified.

  - Work includes all plant facilities, services, materials, labor, tools, and appliances required.

- REFERENCES

  - ANSI A108/A118/A136.1 – American National Specifications for the Installation of Ceramic Tile - A Collection of 20 ANSI/CTI A108 Series Standards on Ceramic Tile Installation: A108.1A-C, 108.4 -.13, A118.1-.10, ANSI A136.1

  - ANSI A137.1 – American National Standards Specifications for Ceramic Tile

  - ASTM C144 - Standard Specification for Aggregate for Masonry Mortar

  - ASTM C206 - Standard Specification for Finishing Hydrated Lime

  - ASTM C207 - Standard Specification for Hydrated Lime for Masonry Purposes

  - TCNA - (Tile Council of North America) - Handbook for Ceramic Tile Installation

- JOB CONDITIONS

  - This Contractor shall inspect the job conditions before starting.

  - All flooring variations less than 1/8” in 10 ft., chips, and cracks are the responsibility of the flooring subcontractor to feather/patch prior to the installation of tile.

- SUBMITTALS

  - Submit manufacturer’s detailed technical data and tile and grout samples per Section 01 33 00.

  - Product Data:  Provide instruction for using adhesives and grouts.

  - Shop Drawings:  Indicate tile layout, patterns, color arrangement, perimeter conditions, and junctions with dissimilar materials, control and expansion joints, thresholds, and setting details.

  - Manufacturer's Certificate:  Certify that products meet or exceed ANSI A137.1.

- EXTRA MATERIALS

  - Furnish extra materials described below that match products installed, packaged with protective covering for storage, and identified with labels describing contents.

  - Furnish quantity of full-size units equal to 2.0% of amount installed.

PART 2	PRODUCTS

- CERAMIC TILE (CT)

  - Tile Products manufactured by American Olean, Dal-Tile, Inter-Ceramic, or US Tile are acceptable, providing their products equal or exceed the type and quality of the specified products and the colors match the colors specified.

  - Provide standard grade ceramic glazed wall tile conforming to ANSI A137.1.

  - Provide 5/16" thick standard size as shown on plans with matte glazed tile and cushioned edge.

    - Use master-set, back mounted sheets.

  - Provide standard grade ceramic unglazed floor tile conforming to ANSI A137.1.

    - Tile shall meet or exceed the ADAAG slip resistance coefficient of 0.6 on static surfaces and 0.8 on ramps, and submit documentation to Architect.

    - Provide 5/16" thick standard size as shown on plans with matte unglazed tile and cushioned edge.

    - Use master-set, back mounted sheets.

  - Glazed Wall Tile Trim, provide the following:

    - Trim in a size, color, and shade to match field tile.

    - Bull nose wainscot cap where required.

    - In a standard, square top, cove base at tile floors.

    - In a square top, set-on type, cove base at other floors.

    - Square edges at inside corners.

    - Bull nose edges at outside corners and jambs.

- CERAMIC MOSAIC TILE (CMT)

  - Tile Products manufactured by American Olean, Dal-Tile, Inter-Ceramic, or US Tile are acceptable, providing their products equal or exceed the type and quality of the specified products and the colors match the colors specified.

    - Tile shall meet or exceed the ADAAG slip resistance coefficient of 0.6 on static surfaces and 0.8 on ramps, and submit documentation to Architect.

  - Provide standard grade ceramic mosaics conforming to ANSI A137.1.

  - Provide rectangular by ¼" thick porcelain type with all purpose edges and patterns.

  - Use master-set, back mounted sheets.

  - Furnish with 7½% abrasive grain content in wet areas only.

- QUARRY TILE

  - Use domestic quarry tile for floor and base, standard grade, slip resistant, in accordance with the minimum grade guidelines established by the TCNA.

  - Tile Products manufactured by American Olean, Daltile, Interceramic Inc, or USA Tile & Marble are acceptable.

    - Tile shall meet or exceed the ADAAG slip resistance coefficient of 0.6 on static surfaces and 0.8 on ramps, and submit documentation to Architect.

    - Floor Tile: Provide and install 6" x 6" x ½" “Quarry Naturals” conforming to ANSI 137.1.

    - Base shall be cove, bull nose top 6" x 5" x ½" to match floor.

- MATERIALS

  - Anti-Fracture Membrane/Cleavage Membrane:  As indicated on the drawings, and elsewhere as required for isolating the installation from cracking due to minor substrate movement and normal structural deflections.

  - Waterproofing and Anti-Fracture Membrane:  As indicated on the drawings, and elsewhere as required for thin-set tile installations complying with ANSI 118.10 for waterproof membranes.

  - Sound Control/Acoustical Underlayment: As indicated on the drawings, and elsewhere as required to be load bearing, shock, and vibration resistant.

  - Moisture Barrier System: As indicated on the drawings and elsewhere as required for thin-set tile installations.

  - Self-Leveling Underlayment: As indicated on the drawings, and elsewhere as required to provide a flat, level surface for direct receipt of tile and other floor coverings on dry, interior installations.

  - Mortar Bed Installations: As indicated on the drawings, and elsewhere as required for mortar bed or brown coat as the substrate for tile work; work to conform to ANSI A108.1.

  - Cementitious Backer Units: ANSI A118.9, where indicated on the drawings, if so; and elsewhere as required for floors wet areas and dry as recommended substrate for tile,  heat shield with UL listing for floors; installation to comply with ANSI A108.11 and manufacturer's installation instructions.

    - ½” Wonder Board Backer board (Exterior or Interior Floors, and Countertops)

  - Use water resistant wallboard as specified in section 09 26 00 for walls and ceilings as indicated on drawings.

  - Cementitious Tile Adhesives:

    - ANSI A118.1: As indicated on the drawings, and elsewhere as required for setting tile as specified by ANSI A108.5, Dry-Set Portland Cement Mortar or Latex Portland Cement Mortar, over substrates prepared accordingly.

    - ANSI A118.4: Polymer-Enhanced Mortars:

    - Latex Additives: As specified, use a latex additive as the mixing liquid, per manufacturer's direction, with certain pre-packaged, dry-set mortar mixes, to achieve a Latex Portland Cement Dry Set Mortar complying with ANSI A118.4.

  - Organic Tile Adhesives:

    - ANSI A136.1: As indicated on the drawings, and elsewhere as required for setting tile as specified by ANSI 108.4, Organic Adhesives, over substrates prepared accordingly.

  - Epoxy Tile Adhesives:

    - ANSI A118.3: As indicated on the drawings, and elsewhere as required for setting tile as specified by ANSI A108.6 Chemical Resistant, Water-Cleanable Tile Setting and Grouting Epoxy, over substrates prepared accordingly.

  - Grout: As indicated on the drawings, and elsewhere as required for filling the joints between tiles.

    - Polymer-Modified Portland Cement Grout:

    - Dry-Set Grout:  Use grout meeting ANSI A118.6, for joints up to 1/8”.

    - Chemical Resistant, As indicated on the drawing and elsewhere as required provide Water-Cleanable Tile Setting and Grouting Epoxy; per ANSI A118.3.

    - Elastomeric Joint Caulk: ANSI A108.01.3.7 As indicated on the drawings, and elsewhere as required provide in all joints between floors and walls and at joints between tile and dissimilar materials.

- ACCESSORIES

  - The Contractor to supply all necessary base, cap edge corner, trim, or accessory tiles required for a complete installation.

  - Shower Pan Membrane (use at shower and drying rooms)

    - Mud-Set Method: Shall be equal to “Chloraloy” Shower Pan Liner chlorinated polyethylene shower pan membrane by The Noble Co.

    - Thin-Set Method: Shall be equal to “Nobleseal TS” Thin-Bed Waterproofing chlorinated polyethylene sheet membrane as manufactured by The Noble Co.

  - Use marble thresholds at all changes of materials, maximum of ½” thick to meet handicap requirements.

  - Sealer – after cleaning, apply two coats of joint sealer per manufacturer’s recommendations.

    - Aqua Mix Grout Sealer

    - Penetrating Sealer – for Quarry and other Vitreous, Semi-Vitreous & Non – Vitreous Tile

    - Other product equal to the above

  - Install layout, pattern, and colors as designed by the Architect.

    - Primary Color – approx. 55%

    - Secondary Color – approx. 25%

    - Tertiary Color – approx. 15%

    - Accent Colors – approx. 5%

PART 3	EXECUTION

- INSPECTION

  - This Contractor shall notify the General Contractor when he has completed his work and is ready for inspection.

- INSTALLATION

  - Install tile, per TCNA F113sloping to floor drains, where applicable, in accordance with ANSI specification A-108.5.

    - Install wall tile per TCNA W202.

  - Install tile in grid pattern with continuous and properly aligned joints in all directions.

  - Install thresholds at all wall openings where quarry tile abuts other floor finishes.

    - Provide maximum threshold height ¼" for HDCP requirements.

    - A threshold up to ½" in height maybe installed, if it has edges beveled at 1:2 slope.

  - Provide a ⅛" joint between the tile, and the doorframe or other item of dissimilar material then use a sealant over the joint instead of grout.

    - Caulk the joints with caulking compound that matches grout.

  - Space tiles a minimum of ½-tile width from corners and edges.

  - Use masonry saw to cut tile unless Architect and Owner approve another method.

  - Install accent wall tile level and true to line and flush with stucco surface.

  - Floor tile in toilet rooms shall be epoxy grouted.

- ADJUSTMENT AND CLEANING:

  - Upon completion of the building, all finishes shall be free of all mars, dents, or other visible imperfections.

- WARRANTY:

  - This Contractor shall fully guarantee all materials and labor under this section for a period of not less than 1-year from date of final acceptance of the building against all defects in both workmanship and materials, and he shall promptly correct and/or replace such faulty work if so notified.

END OF SECTION