# SECTION 09 84 33

ACOUSTICAL WALL PANELS

PART 1	GENERAL

- RELATED DOCUMENTS

  - Drawings and general provisions of Contract, including General and Supplementary Conditions and Division 1 specification sections, apply to work in this section.

- DESCRIPTION OF WORK

  - Provide all labor, materials, and equipment as required to provide the acoustical wall panels as shown on the drawings.

- QUALITY ASSURANCE

  - All packages shall be clearly marked and labeled at plant with manufacturer's name and indicate the specific materials specified.

- REFERENCES

  - ASTM C423 - Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method

  - ASTM E84 - Standard Test Method for Surface Burning Characteristics of Building Materials

  - ASTM E90 Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements

  - UL 723 Tests for Surface Burning Characteristics of Building Materials

- SUBMITTALS

  - Submit complete specifications and installation instructions for all materials furnished.

  - Submit full range of color samples for Architect's selection.

  - Submit shop drawings showing materials, installation details, sizes, and layout.

- DELIVERY, STORAGE & HANDLING

  - Handle material per manufacturer's requirements.

  - Store the materials in the original packaging material in a clean dry area until ready for installation.

  - Do not install until cleanliness and temperature is controllable.

- WARRANTY

  - Provide written warranty, executed by the manufacturer, agreeing to replace defected products for one year after owner's acceptance of installation.

- MAINTENANCE

  - Provide owner with manufacturer's recommendations for cleaning and repairing panels.

PART 2	PRODUCTS

- INTERIOR ACOUSTICAL WALL PANELS:

  - Act (absorptive) Respond wall panels as manufactured by Conwed, Ladysmith Wisconsin (Owens Corning Company) or equal products by

    - AVL System, Inc, FL

    - Armstrong

    - G & S – Golterman & Sabo, MO

    - ESSI Acoustical Products, OH

    - MPC, Inc, OH

    - Lamvin Inc., CA

    - MBI Products Company, Inc, OH

    - Other Pre-approved manufacturer

  - Provide in size per drawings with fiberglass core material (6 PCF) and minimum NRC of 0.80 per inch.

  - Panels shall have beveled edges and be furnished with continuous chemically hardened internal edge protection.

  - Provide woven polyester fabric covering the face, all edges, and a return on the back of a minimum of 1½".

  - All components utilized in the construction of these wall panels shall meet Class A (0-25) rating per ASTM E84, fuel contribution of 20, and smoke density value of 65.

  - Architect will select colors from manufacturer's full range in coordination with the School District Design Coordinator.

- EXTERIOR ACOUSTICAL WALL PANELS

  - Acoustiblok, Inc. 6900 Interbay Blvd., Tampa, outdoor sound absorption panels or equal products by:

    - Acoustical Solutions, Inc., Acoustifiber, Virginia

    - Acoustical Surfaces, Inc., Chaska, MN, Sound Fighter ® Systems Wall

    - Unger Technologies Sound Barrier, Indiana

  - Size per drawing x 2.4” thick

  - All Weather sound panels – hurricane model.

  - Fire Performance, UL 723

  - NRC not less than 0.8

  - Refer to the current edition of the School District Design Criteria for Sound Ratings (http://www.palmbeachschools.org/buildingdepartment/DistrictDesignStandards.asp).

  - Wind resistance not less than 270 mph.

  - Provide framed construction, of corrosion resistant aluminum, mill finish.

# PART 3	EXECUTION

# INSTALLATION:

# Complete installation and workmanship for each type of acoustical system specified under this section in strict accordance with the manufacturers' standard printed specification, recommendations and as detailed on the drawings.

# The wall panels shall be mounted utilizing concealed splines.

# PROTECTION OF FINISHED WORK

# Protect finished work under provisions of Section 01 50 00.

END OF SECTION