SECTION 10 21 23

CUBICLE CURTAINS

PART 1	GENERAL

- RELATED DOCUMENTS

  - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division-1 specification section, apply to work of this section.

- SECTION INCLUDES

  - Overhead metal curtain track and guides

  - Curtains

- REFERENCES

  - ASTM E84 – Standard Test Method for Surface Burning Characteristics of Building Materials

  - NFPA 701 - Fire Tests for Flame Resistant Textiles and Films

  - UL (Underwriters Laboratories, Inc.) - Flammability Test 214

- SYSTEM DESCRIPTION

  - Track:  Surface mounted.

- PERFORMANCE REQUIREMENTS

  - Track:  Support vertical test load of 50 lbs without visible deflection of track or damage to supports or ceilings.

  - Track Size:  Safely support moving loads.

  - Track and Mounting:  Sufficiently rigid to resist visible deflection and without permanent set.

- SUBMITTALS

  - Submit under provisions of Section 01 33 00.

  - Provide shop drawings indicating a reflected ceiling plan view of curtain track, hangers and support points, attachment details and schedule of curtain sizes.

  - Product Data:  Provide data for curtain fabric characteristics and flame-smoke rating.

  - Samples:  Submit two fabric samples illustrating fabric color.

  - Submit 12" x 12" sample patch of curtain cloth with representative hemstitch detail, heading with reinforcement, and carrier attachment to curtain header.

  - Submit 12" sample length of curtain track including typical splice and wall and ceiling hanger and escutcheon.

  - Manufacturer's Installation Instructions:  Indicate special procedures, perimeter conditions requiring special attention.

- MAINTENANCE DATA

  - Submit under provisions of Section 01 77 00.

  - Maintenance Data:  Include recommended cleaning methods and materials and stain removal methods.  Note cleaning methods deleterious to maintaining low flammability standards.

- REGULATORY REQUIREMENTS

  - Conform to applicable code and NFPA 701 for flame/smoke rating requirements in accordance with ASTM E84 for curtain fabric.

- DELIVERY, STORAGE, AND HANDLING

  - Deliver, store, protect, and handle products to site under provisions of Section 01 60 00.

  - Accept curtain materials on site and inspect for damage.

- FIELD MEASUREMENTS

  - Verify that field measurements are as indicated on shop drawings.

PART 2	PRODUCTS

- TRACK MATERIALS

  - Provide a track of extruded aluminum sections one piece per cubicle track run.

  - Track ends stops, tees, and Y's switches to fit track section.

  - Wand is an aluminum hollow section, attached to lead carrier, for pull-to-close action.

  - Provide clear satin anodized finish exposed track surfaces.

- CURTAIN MATERIALS

  - Provide a closed weave nylon curtain with, anti-bacterial, self-deodorizing, sanitized, preshrunk, and flame proofed to UL 214 or NFPA 701.

  - Color selected from manufacturer's standard range by Architect/District Coordinator.

  - Provide open mesh cloth with a fully edge bound, open weave to permit air circulation; flameproof material, of the same color as curtain, 18" to 24" width at top of curtain.

- CARRIERS AND HARDWARE

  - Nylon axle and wheel carriers with nickel-plated brass beaded chain and polished aluminum hook.

  - Provide curtain carriers of nylon roller accurately fitting the track; designed to eliminate bind when curtain is pulled, fitted to curtain to prevent accidental curtain removal.

- FABRICATION

  - Manufacture curtains of one piece, sized 10% wider than track length.

    - Terminate curtain 15" from floor.

  - Include open mesh cloth at top of curtain for room air circulation.

  - Curtain Heading:

    - Triple thickness 2" wide, with metal grommet holes for carriers at 6" oc; double fold bottom hem 2" wide included lead weights.

    - Lock stitch seams in two rows.

    - Turn seam edges and lock stitch.

  - Fabricate track bend with minimum 12" radius, without deforming track section, or impeding movement of carriers.

PART 3	EXECUTION

- EXAMINATION

  - Verify that surfaces and above ceiling supports are ready to receive work.

  - Do not anchor track to finish ceiling unless properly designed to accommodate stresses encountered during curtain operation.

- INSTALLATION

  - Install curtain track secure and rigid, true to ceiling line.

  - Install end cap and stop device.

  - Secure track to ceiling system.

  - Install curtains on carriers ensuring smooth operation.

END OF SECTION