SECTION 10 14 33

EXTERIOR MARQUEE SIGN

PART 1	GENERAL

- RELATED DOCUMENTS

  - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division-1 specification section, apply to work of this section.

- SECTION INCLUDES

  - Cabinet, face, and message area of exterior marquee sign specifications.

  - Lighting

  - Structural support

- REFERENCES

  - ASCE 7 – Minimum Design Loads for Buildings and Other Structures

  - ASTM A36/A36M - Standard Specification for Carbon Structural Steel

  - ASTM A307 - Standard Specification for Carbon Steel Bolts and Studs, 60,000 PSI Tensile Strength

  - ASTM A500/A500M - Standard Specification for Cold-Formed Welded and Seamless Carbon Structural Tubing in Rounds and Shapes

  - ASTM A615/A615M - Standard Specification for Deformed and Plain Carbon Steel Bars for Concrete Reinforcement

  - ASTM C33/C33M - Standard Specification for Concrete Aggregates

  - ASTM C150/C150M - Standard Specification for Portland Cement

  - ASTM C270 Standard Specification for Mortar for Unit Masonry

  - FBC - Florida Building Code

  - NEMA Standards

  - UL Underwriters Laboratories, Inc

- SUBMITTALS

  - Submit shop drawings under provisions of Section 01 33 00.

  - Shop Drawings:  Indicate sign styles, lettering font, foreground and background colors, locations, overall dimensions of each sign and anchorage.

    - Anchorage shall include as a minimum an engineered foundation plans/details, structural components and connections to each other, the sign cabinet, and the foundation.

  - Colors:  Colors shall be as selected by the Architect.

- QUALIFICATIONS

  - Manufacturer:  Company specializing in manufacturing the products specified in this section with minimum three years documented experience.

Design Work under direct supervision of a Professional Structural Engineer experienced in design of this work and licensed in the State of Florida.

- REGULATORY REQUIREMENTS

  - Conform to FBC and ASCE-7.

- WARRANTY

  - Warranty for 10-years against defects in workmanship and materials, the warranty does not cover lamps or damage from gunshots.

- DELIVERY, STORAGE, AND HANDLING

  - Deliver, store, protect, and handle products to site per manufactures requirements.

# PART 2	PRODUCTS

- MATERIALS

  - Cabinet Specifications

    - Double sided extruded aluminum and sized as indicated on plans

    - Heliarc welded at mitered at corners

    - Powder coated color finish: 3.0 to 4.0 mils thick electrostatically applied and permanently bonded guaranteed non-fading.

  - Cabinet Face Specifications

    - Provide vandal proof clear UV solar grade high impact resistant polycarbonate cover with stainless steel hinges secured with dual compression tubular locks.

    - Powder coat covers to match cabinet.

  - Message Area Specifications

    - Provide a Marquee Inner Face System using 3M white translucent acrylic with a chemically bonded clear letter track or approved equal.

    - Provide minimum 500 count self-spacing letters and numbers.

      - Double silk screened on 1/8"injected molded substrate.

      - Provide in a font as selected by Architect and District Design Coordinator.

      - Provide letter storage case of durable material, i.e. impact resistant plastic.

      - Provide all letters and numbers in black.

  - Lighting Specifications

    - All electrical components shall be UL listed and approved.

    - Lighting: High output instant start fluorescent tubes spaced to provide uniform night lighting over entire face of the sign.

    - Install per current NEC edition.

  - Support Structure Specifications

    - Design the support structure to withstand wind loads of as required by ASCE 7 & FBC.

    - Legs: Cover on both sides if indicated on plans and anchor to foundation as indicated on engineered plans.

    - Side frames painted in color approved by the Architect and District Design Coordinator.

    - Provide signed and sealed (by Florida licensed PE) engineered shop drawings and structural calculations for approval by District Building Department.

  - Provide 6” high numbers on the marquee, both sides if they are viewable from the street.

PART 3	EXECUTION

- INSTALLATION

  - Install signs in strict accordance with manufacturer's instructions and FBC requirements.

- CLEANING

  - After installation, thoroughly clean all exposed surfaces and restore all damaged material to its original condition or replaced with new material.

END OF SECTION