SECTION 10 22 13

WIRE MESH/CHAIN LINK PARTITIONS

PART 1	GENERAL

- RELATED DOCUMENTS

  - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 specification section, apply to work of this section.

  - Section  32 31 13 Chain Link Fencing & Gates

- SECTION INCLUDES

  - Wire mesh or chain link partitions and systems for walls

  - Access door/gate

- REFERENCES

  - ASTM A36/A36M - Standard Specification for Carbon Structural Steel

  - ASTM A123/A123M – Standard Specification for Zinc (Hot-Dipped Galvanized) Coatings on Iron and Steel Product

  - ASTM A167 – Standard Specification for Stainless and Heat-Resisting Chromium-Nickel Steel Plate, Sheet and Strip

  - ASTM A500/A500M – Standard Specification for Cold-formed Welded and Seamless Carbon Steel Structural Tubing in Rounds and Shapes

  - ASTM A501 – Standard Specification for Hot-formed Welded and Seamless Carbon Steel Structural Tubing

  - ASTM A510/A510M – Standard Specification for General Requirements for Wire Rods and Course Round Wire, Carbon Steel

  - ASTM A580/A580M – Standard Specification for Stainless Steel Wire

  - ASTM A653/A653M – Standard Specification Sheet Steel, Zinc-Coated (Galvanized) by the Hot Dip Process, Structural or Zinc-Iron Alloy-Coated (Galvannealed)

  - ASTM A794/A794M - Standard Specification for Commercial Steel (CS), Carbon (0.16% Maximum to 0.25% Maximum), Cold Rolled

  - AWS D1.1 - Structural Welding Code

  - SSPC (Steel Structures Painting Council) - Painting Manual

- DESIGN REQUIREMENTS

  - Design partitions system to provide for movement of components without damage, undue stress on fasteners or other detrimental effects, when subject to design loads.

  - Design system to accommodate construction tolerances, deflection of building structural members, and clearance of intended openings.

  - Design partitions to resist the anticipated or potential lateral loads but not less than 10 psf.

  - Design gates with locking mechanism that allows egress from a partitioned area at any time.

  - Increase framing sizes for partitions exceeding 12'0" in height.

- SUBMITTALS

  - Submit under provisions of Section 01 33 00.

  - Shop Drawings:

    - Indicate plan and vertical dimensions, elevations, component details; head, jamb, and sill details; location of hardware.

    - Provide component details, framed openings, bearing, anchorage, loading, welds, type, and location of fasteners, and accessories or items required of related work.

  - Product Data:  Provide data for screen materials and finishes.

    - Any expanded metal used for partitions must be completely smooth and free of sharp edges.

  - Samples:

    - Submit two samples, illustrating screen material.

    - Submit samples of hinge, latch set illustrating style, color, and finish.

    - Incorporate sample into the work.

  - Manufacturer's Installation Instructions:  Indicate special procedure, perimeter conditions requiring special attention.

- QUALIFICATIONS

  - Manufacturer:

  - Company specializing in the manufacturing products specified in this section.

  - Minimum 3-years documented experience and complies with the standards of the Woven Wire Products Association.

- FIELD MEASUREMENTS

  - Verify that field measurements are as shown on shop drawings.

PART 2	PRODUCTS

- MATERIALS

  - Framing Members meeting the requirements of ASTM A36/A36M formed steel sections.

  - Woven Wire Screen:  ASTM A510/A510M uncoated crimped steel wires; conforming to the following:

    - Warp and Fill Wire Size:  10-gauge (.135”)

    - Mesh Size:  Maximum 2 x 2 inch

    - Mesh Weave Design:  Plain weave, inter-crimp design, diamond

    - Channel framing: Minimum 1/8" thick

    - Maximum 5'0" panel width

  - Provide welding materials meeting the requirements of AWS D1.1.

  - See Section 32 31 13 Chain Link Fencing & Gates for requirements on galvanized chain link.

- FASTENERS

  - Bolts, Nuts and Washers shall be unfinished, or as recommended by manufacturers.

  - Provide power driven anchorage devices:

  - Exposed Mechanical Fasteners, provide flush, countersunk screws or bolts unobtrusively located and consistent with design of structure with vandal proof heads for exterior side.

- ACCESSORIES

  - Bracing:  Provide formed or cold-rolled sheet steel, thickness determined for conditions encountered, and same finish as framing members.

  - Plates, Gussets, Clips:  Provide formed sheet steel, thickness determined for conditions encountered, manufacturer's standard shapes, same finish as framing members.

  - Capping Bar:  Provide minimum 2" x 1" cold-rolled channel.

  - Main Framing:  Provide minimum 1" x ½" x 1/8" cold-rolled on formed channel.

  - Shop and Touch-Up Primer shall be SSPC 15, Type 1, red-oxide.

- DOORS AND HARDWARE

  - Doors:  Doors shall be swing type, 3' x 7' unless otherwise noted, bended on all four sides with minimum 1/8" thick steel bar.

  - Hinges:  Provide the manufacturer’s standard.

  - Latch Sets/Locks:  Mortised with cylinder type lock, exterior keyed and interior egress lever.

- FABRICATION

  - Fit and assemble in largest practical sections for delivery to site, ready for installation.

  - Make exposed joints flush or tight.

  - Provide components required for anchorage to adjacent construction.

  - Frame openings made for penetrating mechanical and electrical components.

- FINISHES

  - Clean surfaces of rust, scale, grease, and foreign matter prior to finishing.

  - Prime the surfaces with one coat, except surfaces where field welding is required.

PART 3	EXECUTION

- EXAMINATION

  - Verify that substrate surfaces and required openings are ready to receive work.

- PREPARATION

  - Clean substrate surfaces.

- INSTALLATION

  - Install in accordance with manufacturer's instructions and approved submittals.

  - Install items plumb and level, accurately fitted, free from distortion or defects.

    - Doors/gates shall operate freely without binding

  - Perform field welding in accordance with AWS D1.1.

  - After installation, touch-up field welds scratched or damaged surfaces with shop-finish.

- ERECTION TOLERANCES

  - The maximum variation from plumb or level is 1/4".

  - The maximum misalignment from True Position is 1/4".

- ADJUSTING

  - Adjust work under provisions of Section 01 77 00.

  - Adjust hinged doors/gates to achieve free movement.

- CLEANING

  - Clean work under provisions of Section 01 77 00.

  - Remove temporary protection to pre-finished surfaces.

END OF SECTION