# SECTION 10 26 00

DOOR and Wall Protection

Part 1	GENERAL

- RELATED DOCUMENTS

  - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division-1 specification section, apply to work of this section.

- SUMMARY

  - This section includes the following types of wall protection systems:

    - Handrails

    - Bumper Guards/Crash Rails

    - Corner Guards

  - Related Sections: The following sections contain requirements related to this section:

    - Blocking in walls for fasteners refer to section 06 10 00 "Rough Carpentry" and section 09 29 00 "Gypsum Board System"

    - Kick plates and door protection; refer to section 08 71 00 "Door Hardware"

- References

  - ASTM B221 – Standard Specification for Aluminum and Aluminum Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes

  - ASTM D523 – Standard Test Method for Specular Gloss

  - ASTM D1308 – Standard Test Method for Effect of Household Chemicals on Clear Pigmented Organic Finishes

  - FBC - Florida Building Code

  - Underwriters Laboratories (UL)

- SUBMITTALS

  - General: Submit the following in accordance with conditions of contract and Division 1 specification section 01 33 00.

  - Product data and detailed specifications for each system component and installation accessory required, including installation methods for each type of substrate.

  - Provide shop drawings showing locations, extent and installation details of handrails, bumper guards, corner guards, and other protection systems.

    - Show methods of attachment to adjoining construction.

  - Submit the following samples, as proposed for this work, for verification of color, texture, pattern and end cap attachment/alignment:

    - One, 12" long sample of each model specified including end cap and mounting hardware.

  - Product test reports from a qualified independent testing laboratory showing compliance of each component with requirements indicated.

  - Maintenance data for wall protection system components for inclusion in the operating and maintenance manuals specified in Division 1.

- QUALITY ASSURANCE

  - Installer qualifications:  Engage an installer who has not less than 3-years of experience in installation of systems similar in complexity to those required for this project.

  - Manufacturer’s qualifications:  Shall be not less than 5-years of experience in the production of specified products and a record of successful in-service performance.

  - Code compliance: Assemblies shall conform to all applicable and referenced codes.

  - Fire performance characteristics: Provide wall protection system components with a UL label indicating that they are identical to those tested in accordance with ASTM- E84for Class 1 characteristics listed below:

    - Flame spread index of 25 or less

    - Smoke developed index of 450 or less

  - Impact Strength: Provide assembled wall protection units tested in accordance with the applicable provisions of FBC 1608.

  - Color match, unless otherwise indicated, wall protection components.

  - Single source responsibility: Provide all components of the wall protection system manufactured by the same company to ensure compatibility of color, texture, and physical properties.

- DELIVERY, STORAGE, AND HANDLING

  - Deliver materials to the project site in unopened original factory packaging clearly labeled to show manufacturer.

  - Store the materials in original, undamaged packaging in a cool, dry place out of direct sunlight and exposure to the elements.

    - Maintain a minimum room temperature of 40° F and max of 100° F.

- PROJECT CONDITIONS

  - Environmental requirements: Installation areas must be enclosed and weatherproof before installation commences.

  - Maintain ambient temperature above 65° F during, and for 24 hours after installation.

Part 2	Products

- MANUFACTURERS

  - Provide Acrovyn from Construction Specialties Inc. unless otherwise indicated.

  - Other manufacturer’s products complying with the minimum levels of material and detailing indicated on the drawings and specified herein shall be acceptable.

- MATERIALS

  - Vinyl/Acrylic:

    - Extruded material shall be high impact with pebblette grain texture and thickness as indicated on plans.

    - Chemical and stain resistance shall be per ASTM D1308 standards.

    - Colors are as indicated in the finish schedule from manufacturer's standard color range.

  - Extruded aluminum retainers should be 6063-T6 alloy of thickness indicated, with a minimum strength and durability properties as specified in ASTM B221.

  - Fasteners:

    - Provide non-corrosive fasteners compatible with aluminum retainers.

    - Manufacturer shall supply all necessary fasteners.

- HANDRAILS

  - Vinyl/Acrylic handrails:

    - Use a surface-mounted assembly consisting of a continuous extruded aluminum retainer with snap-on cover.

    - Include integral shock absorbing cushions where indicated.

    - Provide handgrip size and wall standoff in compliance with accessibility codes.

    - Color matched mounting brackets to be spaced as indicated on installation instructions.

    - Attach color matched end caps and corners to allow full post-installation adjustment.

    - Attachment hardware shall be appropriate for wall construction.

- BUMPER GUARDS/CRASH RAILS

  - Bumper guards and crash rails:

    - Provide surface-mounted assembly consisting of continuous extruded aluminum retainer with snap-on cover.

    - Include integral shock absorbing cushions where indicated.

    - Space mounting cushions and extension brackets as indicated on installation instructions.

    - Color matched end caps to be removable for ease of replacement, the attachment hardware shall be appropriate for wall construction.

- ACCENT RAILS/RETAIL BUMPERS

  - Accent rails:

    - Provide surface-mounted assembly consisting of a continuous retainer with snap-on cover.

    - Permanently affix retainer to wall at 16" o.c.

    - Mechanically fasten color matched end caps with concealed fasteners.

  - Bumper guards:

    - Provide surface-mounted assembly consisting of continuous co-extruded rigid vinyl retainer, flexible hinge.

    - All profiles offer the integral shock-absorbing feature to protect wall substrates or fixture equipment.

    - Color matched end caps to be removable for ease of replacement.

    - Attachment hardware shall be compatible for wall construction.

- CORNER GUARDS

  - Surface-mounted corner guards.

    - Surface-mounted guards consisting of continuous aluminum retainer (optional PVC where indicated) with snap-on cover.

    - Provide color matched end caps for both partial and full height applications.

    - Attachment hardware shall be appropriate for wall construction.

  - Flush-mounted corner guards:

    - Flush-mounted guards consisting of continuous aluminum retainer with snap-on cover.

    - Installer shall tape and spackle the retainer into the adjacent drywall.

    - Terminator caps provided for partial height and soffit applications to be taped and spackled into adjacent drywall.  Install retainer from floor to 2" above finished ceiling.

    - Extruded aluminum base and closure gasket where indicated.

    - Attachment hardware shall be appropriate for wall construction.

- ACCESSORIES

  - Size all mounting hardware for required load and substrate conditions.

- FABRICATION

  - Fabricate wall protection systems to comply with requirements indicated for design, dimensions, detail, finish, and member sizes.

  - Preassemble components in the shop to the greatest extent possible to minimize field assembly.

  - Fabricate components with tight seams and joints with exposed edges rolled.

    - Provide surfaces free of chipping, dents, and other imperfections.

- FINISHES

  - Comply with NAAMM "Metal Finishes Manual" for recommendations relative to applications and designations of finishes.

  - Aluminum mill finish shall be AA-MIO.

  - Wood Finish shall be AWI 60-degree gloss per ASTM D523.

Part 3	Execution

- EXAMINATION

  - Verification of conditions:

    - Examine areas and conditions of work area and identify conditions detrimental to proper or timely completion.

    - Do not proceed with work until unsatisfactory conditions are corrected.

- PREPARATION

  - Prepare surface prior to installation, clean substrate to remove dirt, debris, and loose particles, and perform additional preparation procedures as required by manufacturer’s instructions.

  - Take all necessary steps to prevent damage to material during installation as required in manufacturer’s installation instructions.

- INSTALLATION

  - Install the work of this section in strict accordance with the manufacturer’s recommendations, using only approved mounting hardware, and locating all components firmly into position, level, and plumb.

  - Adjust installed end caps as necessary to ensure tight seams.

  - Where splices occur in horizontal runs of over 20-feet, provide splice aluminum retainer and vinyl cover at different locations along the run.

- CLEANING

  - Immediately upon completion of installation, clean vinyl covers and accessories in accordance with manufacturer’s recommended cleaning method.

  - Remove surplus materials, rubbish, and debris resulting from installation as work progresses and upon completion of work.

- PROTECTION

  - Protect installed materials to prevent damage by other trades.

  - Use easily removable materials that do not leaving residue or permanent stains.

END OF SECTION