SECTION 11 51 23

LIBRARY STACK SYSTEMS

PART 1	GENERAL

- RELATED DOCUMENTS

  - The provisions of the General Conditions, Supplementary Conditions, and the Sections included under Division 1, General Requirements, are included as a part of this Section.

- SECTION INCLUDES

  - This specification covers materials of the library steel, bracket type book-stack.

    - Unit heights, depths, and accessories as indicated on floor plans, specifications, or schedule of equipment.

    - Provide proper floor anchoring for double-faced book stack and wall anchoring for wall or single faced shelving in an inconspicuous manner.

    - Determine anchor requirements by field conditions in accordance with local codes and shelving manufactures requirements, otherwise not included in this specification.

    - Provide Library Bureau SafeStak or Architect approved equal.

- SUBMITTALS

  - Manufacturer’s descriptive product data

  - Shop drawings showing shelving layout

- WARRANTY

  - Provide manufacturer’s warranty defects in materials and workmanship for 1-year from the date of Substantial Completion.

PART 2	PRODUCTS

- DESCRIPTION

  - Sway brace construction shall be of the bracket type, double-faced or single-faced with closed bases.

- MATERIALS

  - Sheet metal to be class-1 cold or hot rolled and full pickled.

  - Gauge thicknesses are U.S. standard with the following minimum requirements:

    - Shelves – 18-gauge

    - Web stiffeners – 13-gauge

    - Base brackets – 13-gauge

    - Top tie channel – 16-gauge

    - Adjustable shelf brackets – 16-gauge

    - Upright columns – 16-gauge

- CAPACITY REQUIREMENTS

  - Each shelf shall have a minimum clearance between brackets of 35-7/16".

- FINISH

  - Automatically wash and phosphate coat all parts.

  - Following a complete drying process, each part is to receive a finish coat of high solids polyester blend backed enamel.

- COLOR

  - Color selection is from manufacturer’s standard (unless otherwise specified).

- TYPE OF BOOK STACK

  - Adjustable shelves hang on a central upright column in a cantilever manner allowing a wide flexibility of sizes and accessory arrangement in any standard 36" wide section.

  - Supply sections 24" wide or 30" wide as specified.

  - Following are required component parts.

    - INITIAL UPRIGHT COLUMNS

      - Form of not less than 16-ga. into a channel shape with no less than ½" stiffening flanges measuring 2" in the web and 1-3/16" at front and rear surfaces.

      - Uprights are to be perforated with a series of ¼" x ⅝" slots spaced 1" on vertical centers and located within 5/16" from the web.

      - Place every fifth and sixth slot shape differently for ease of shifting adjustable shelves.

      - Design uprights to receive sway braces for maximum stability.

      - All single-faced sections and double-faced sections receive two upright columns.

    - DOUBLE-FACED UPRIGHT ASSEMBLIES

      - Consist of two uprights as described for the initial upright columns plus a 2¼" wide web stiffener of 13-gauge steel spot-welded together.

      - Supply this for units over 66" high; the spot-welds will be approximately 11" apart.

      - The space between the upright columns produced by the web stiffener provides a slip joint for the insertion of the base brackets and tie channel.

      - Units 66" high and less use two upright columns bolted together with ⅜" machine bolts and nuts.

    - TOP TIE CHANNELS

      - Consist of not less than 16 ga Box formed 2-5/32" deep with 1⅜" downward flanges formed 90° ends.

      - Mechanically fasten individual channels to uprights with nuts and bolts.

    - WEB STIFFENERS

      - Shall be 13-ga. 2¼" wide approx. 7½" shorter than uprights on all stacks over 66" high, keyed to engage base brackets to form a continuous structural member from floor to top of stack.

      - Weld web stiffener between two upright columns to form an upright assembly.

    - BASE BRACKETS

      - Design to fit between uprights.

      - Material shall be no less than 13-ga. Brackets to have 90° flange at bottom to rest on floor punched to receive leveling glides and allow for 6 point leveling when specified.

      - Top and front edge of base brackets angled to match adjustable shelf brackets and to allow nesting adjustable shelves into base shelf area when desired.

    - CLOSED BASE SHELVES

      - Form to meet industry performance standards of 50 plf without deflection in excess of 3/16".

      - One-piece construction designed to fit snugly around upright columns and base brackets without need of hardware fasteners.

      - Front height shall be at least 3" and sides shall have stiffening flanges, designed to inter-member with base brackets slots.

    - SWAY BRACES

      - Combined with the welded SafeStak web stiffened column shall be provided as primary keys in meeting regulatory requirements for seismic risk zones established by authority having jurisdiction.

      - Provide all double-faced ranges with diagonal sway braces installed between the uprights, every third section with a minimum of one pair per range to prevent longitudinal sway of the stack ranges.

      - Use min ¼" steel rods sway braces, hooked into the web of stack uprights and provide turnbuckles insuring uprights in stack ranges are vertically plumb.

    - HARDWARE shall be rust proof, cadmium plated or equal and completely concealed from view in finished assembly with no projections to harm material or users.

    - SHELVES AND ACCESSORIES may use all flat, Tu-bar, divider and all special purpose shelves and accessories in any combination on SafeStak/SB.

PART 3	EXECUTION

- INSTALLATION

  - In accordance with manufacturer’s installation procedures and design criteria.

END OF SECTION