- GENERAL

  - SECTION INCLUDES

    - Construction Parking Control.

    - Flagmen

    - Flares and Lights.

    - Haul Routes.

    - Traffic Signs and Signals.

    - Removal

  - SIGNS, SIGNALS AND DEVICES

    - Post-Mounted and Wall-Mounted Traffic Control and Informational Signs as required to maintain adequate standards of safety and control of vehicular movement on and off construction site.

    - Traffic Control Signals:  As approved by local jurisdictions.

    - Traffic Cones and Drums, Flares and Lights:  As approved by local jurisdictions.

    - Flagmen Equipment:  As approved by local jurisdictions.

  - CONSTRUCTION PARKING CONTROL

    - Control vehicular parking to prevent interference with public traffic and parking, access by emergency vehicles and Owner's operations.

    - Monitor parking of construction personnel's vehicles.  Maintain vehicular access to and through parking areas.

    - Prevent parking on or adjacent to access roads or in non-designated areas.

  - FLAGMEN

    - Provide trained and equipped flagmen to regulate traffic when construction operations or traffic encroach on public traffic lanes.

  - FLARES AND LIGHTS

    - Use flares and lights during hours of low visibility to delineate traffic lanes and to guide traffic.

  - HAUL ROUTES

    - Consult with authority having jurisdiction in establishing public thoroughfares to be used for haul routes and site access.

    - Confine construction traffic to designated haul routes.

    - Provide traffic control at critical areas of haul routes to regulate traffic, to minimize interference with public traffic.

  - TRAFFIC SIGNS AND SIGNALS

    - At approaches to site and on site, install at crossroads, detours, parking areas and elsewhere as needed to direct construction and affected public traffic.

    - Install and operate traffic control devices as are necessary to direct and maintain orderly flow of traffic in areas under Contractor's control and areas affected by Contractor's operations.

    - Relocate as Work progresses to maintain effective traffic control.

  - REMOVAL

    - Remove equipment and devices when no longer required.

    - Repair damage caused by installation.

    - Remove post settings to full depth.

- PRODUCTS

NOT USED.

- EXECUTION

NOT USED.

END OF SECTION