Note to OAR: The Hazardous Materials Phase I Report is to be included as Attachment “A” as noted below in 1.01 (B) (4). Delete this text box upon completion of specification preparation.

# PART 1 - GENERAL

# 1.01	SUMMARY

- Provisions of Division 01 apply to this section

- Section Includes:

    - Abatement of all hazardous materials identified in the Hazardous Materials Phase I Assessment Report, except for underground storage tanks, contaminated soil or groundwater. Furnish all labor, materials, supplies, and incidentals required, protect all Project site personnel and the surrounding public from exposure to potentially hazardous substances, and prevent the spread of potentially contaminated or hazardous substances.

2.	The furnishing of all labor, supervision, materials, equipment, tools, permits, manifesting, and services required in the characterization, transportation, and disposal of identified or suspected hazardous substances.  The suspect substance may be in drums, containers, stockpiled, or may exist as debris piles on the Project site.

- Related Sections:

- Section 02 41 00: Site Cleaning and Demolition

- Section 02 82 00: Asbestos Abatement

- Section 02 83 00: Lead Abatement and Lead Related Construction Work.

- Attachment A – Hazardous Materials Phase I Assessment Report

D.	Regulatory Requirements shall include, but not be limited to: Comply with all laws, ordinances, codes, rules, and regulations of the Federal, State and local authorities having jurisdiction over any of the Work specified herein. Comply with all federal EPA and state Department of Transportation regulations for shipping of hazardous substances to offsite disposal facilities.  Comply with any regulatory requirements imposed by the treatment, storage, and disposal facility.  Regulations pertaining to the transport and disposal of hazardous substances/materials include, but are not limited to, the following:

    - Department of Transportation 49 CFR 172 through 179.

    - Department of Transportation 49 CFR 387 (46 FR 30974, 47073).

    - Department of Transportation DOT-E 8876.

    - Environmental Protection Agency 40 CFR 136 (41 FR 52779).

    - Environmental Protection Agency 40 CFR 261, 262 and 761.

    - Resource Conservation and Recovery Act (RCRA).

- Any transporter of hazardous substances shall be licensed in the state in which handling and transportation will take place in accordance with all applicable regulations.

- Comply with OSHA (Occupational Safety and Health Administration) Standards and Regulations contained in Title 29 Code of Federal Regulations, Part 1910.120 "Hazardous Waste Operations and Emergency Response."

- Where reference is made to one of the above standards, the revision in effect at the time of bid opening shall apply.

# 1.02	QUALITY ASSURANCE

A.	The Work of this Section shall be performed by an entity possessing the following minimum qualifications:

1.	Contractors License in the state where the Work is performed, supplemented by a Hazardous Waste specialty license, where applicable (i.e., California).

2.	MCS 90 Endorsement on Liability Insurance.

3.	Pollution Liability Insurance in the amount of $2,000,000 occurrence.

4.	40-hour OSHA Training and site-specific health and safety plan for its employees proposed to work at the Project site as defined in 29 CFR 1910.120.

5.	A Comprehensive Quality Assurance Plan on file with the California Environmental Protection Agency.

6.	All Project site personnel shall wear personal protective equipment and protective clothing consistent with the levels of protection required for this Work as specified by OSHA (29 CFR Part 1910.120).

PART 2	- PRODUCTS (NOT USED)

PART 3 -	EXECUTION

  - ABATEMENT OF HAZARDOUS MATERIALS

- Abate all hazardous materials including hazardous materials identified in the Phase I Assessment Report.

B.	The Work of this section does not include the removal of underground storage tanks, contaminated soil or groundwater, unless specifically authorized in writing by the OAR.

3.02	CONTAINMENT OF RELEASED SUBSTANCES

A.	If a hazardous material appears to be leaking or otherwise spreading, contain the release of the material.  Provide all measures to prevent the release of the material to the environment and protect all Project site personnel, adjacent properties and occupants, and the general public from potential exposure.

- During the course of substance containment or evacuation of Project site personnel, protect personnel (onsite workers, non-workers, or the general public) from contact with or exposure to the released substances.

- The abatement or evaluation of any suspected hazardous material shall only be performed by properly trained and/or certified personnel.

3.03	HEALTH AND SAFETY

A.	In an emergency or imminent hazard situation the health and safety of all personnel on or near the Project site are the responsibility of the Work of this section. Immediately notify the Owner office of Environmental Health and Safety (OEHS) and the OAR designated representative of the Owner Consultant. Owner representatives will notify Federal, State and Local regulatory authorities, if required.

B.	Any Project site personnel or members of the general public who have been exposed to or have come in contact with any hazardous materials or chemicals shall be immediately transported to the nearest hospital.

3.04	TESTING, TRANSPORT, AND DISPOSAL OF HAZARDOUS SUBSTANCES

- Collect one composite sample from each media type (solid, liquid, or sludge) of potentially hazardous substance stored in drums, stockpiled, or otherwise identified at the Project site for the purposes of obtaining approvals for proper transport and disposal of the suspect materials.  Submit all analytical results to a representative of the Owner Consultant, as designated by the OAR.

- If required, overpack any leaking or deteriorated drums to prevent leaks or spills, and pack small 5-gallon containers into larger new 55-gallon drums.  Cover all solid waste materials and stockpiled soils with an HDPE liner to prevent stormwater runoff from contaminating surrounding areas.

- Prepare manifests, material profiles, and submit lab analysis for all drums/containers and any other documentation required by the receiving facility for signature by a representative of the Owner Consultant and/or Owner, as designated by the OAR. Copies of all waste profiles, manifests, and disposal documentation shall be submitted to an OAR designated representative of the Owner, prior to disposal and/or transporting of hazardous substances.

- Coordinate waste sampling and analysis requirements with the disposal facility and properly complete all profiling and transport documents prior to loading and transport.

- A State registered “Hazardous Waste Hauler” shall transport the waste to a lawfully permitted and Owner approved facility.

- Prior to transport, a copy of the hazardous waste manifest shall be transmitted by facsimile, through the Project site Owner Consultant, to the Construction and Facilities Department for review and approval.  The manifests shall list the generator’s name as “Fontana Unified School District” and the mailing address as “9851 Catawba Avenue, Fontana, CA 92335”.  The OAR designated representative of the Owner Consultant shall provide the Project site specific generator’s EPA ID # which shall be included on the manifest.  Attach the land disposal restriction (LDR) form to the manifest prior to submission to the Owner.  The blue and yellow manifest copies and a copy of the LDR form shall be mailed to the Construction and Facilities Department at 9851 Catawba Avenue, Fontana, CA 92335. All other copies of the manifest and LDR shall be submitted to the waste transporter.

- Load, handle, and transport all 55-gallon drums and other waste containers to the appropriate disposal facility in accordance with Federal and State regulations.

- All transport documentation from the receiving facility verifying acceptance and receipt of drums/containers at the facility and all sampling and associated test results shall be submitted to the representative of the Owner Consultant, as designated by the OAR, within fifteen (15) days following receipt of all hazardous substances to the disposal facility.

I.	All materials identified as hazardous wastes under RCRA are not permitted to remain at the Project site more than 30 days after being deemed to be a hazardous waste.  During this period of Project site storage, provide all precautions to contain and prevent the release of hazardous or potentially hazardous materials to the environment.

# END OF SECTION