- GENERAL

  - SECTION INCLUDES

    - Accordion folding door.

    - Ceiling track and operating hardware.

  - REFERENCES

    - ASTM E84 - Surface Burning Characteristics of Building Materials.

    - ASTM E90 - Airborne Sound Transmission Loss of Building Partitions.

    - Chapter 8 - California Building Code.

  - PERFORMANCE REQUIREMENTS

    - Sound Transmission Coefficient (STC):  ASTM E90, STC of 48, tested on panel size of 100 sq ft.

    - Surface Burning of Vinyl Fabric:  ASTM E84; flame spread rating of less than 25.

    - Install partition system track capable of supporting imposed loads with maximum deflection of 1/360 of span.

  - SUBMITTALS

    - Shop Drawings:  Indicate opening sizes, track layout, details of track and required supports, track loads, adjacent construction and finish trim and stacking sizes.

    - Product Data:  Describe partition operation, hardware and accessories, colors and finishes available.

    - Three samples of surface finish illustrating quality, color, texture, weight.

    - Manufacturer's Installation Instructions:  Include specific installation sequence, special instructions.

    - Include recommended cleaning methods, cleaning materials and stain removal methods.

  - QUALIFICATIONS

    - Manufacturer:  Company specializing in manufacturing the products specified in this Section with minimum five years experience.

    - Installer:  Approved by the manufacturer.

  - REGULATORY REQUIREMENTS

    - Conform to Tables 8-A and 8-B, California Building Code for combustibility requirements for materials.  Flame Spread Requirements: 0-25, smoke density no greater than 450.

  - FIELD MEASUREMENTS

    - Verify that field measurements are as indicated on shop drawings.

- PRODUCTS

  - MANUFACTURERS

    - Products of following manufacturers form basis for design and quality intended.

      - Won Door Corp., Salt Lake City, UT.

      - Modernfold, Greenfield, IN.

      - Curtition Inc., Darien, WI.

      - Holcomb and Hoke Manufacturing Inc., Indianapolis, IN.

    - Or equal as approved in accordance with Division 1, General Requirements for substitutions.

  - COMPONENTS

    - Partition Construction:  Vinyl-clad 24 gauge corrugated, steel panels with permanent finish, two parallel walls independently suspended, DURASOUND 48, non-rated, insulated.

    - Track:  Extruded aluminum 1-1/4 inch high, total 12 inches wide, double raceway, white baked enamel.

  - FINISHES

    - Vinyl Laminate Fabric Finish:  Class A Fire Resistant, heavy mildew resistant, polyvinyl fluoride finish for washability and flame retardance; Color:  Refer to Section 09 05 00.

- EXECUTION

  - EXAMINATION

    - Verify that field conditions are acceptable and are ready to receive work.

    - Confirm track supports are laterally braced and will permit track to be level within 1/4 inch of required position and parallel to the floor surface.

    - Confirm floor flatness of 1/8 in 10 feet, non-cumulative.

    - Beginning of installation means installer accepts existing conditions.

  - INSTALLATION

    - Install partition in accordance with manufacturer’s instructions.

    - Fit and align partition assembly level and plumb.

  - ADJUSTING

    - Adjust partition assembly to provide smooth operation from stacked to drawn position.

    - Inspect partition in drawn position for light leaks to identify a potential acoustic leak.  Adjust to achieve light-tight installation.

    - Test all components for operational performance.

  - CLEANING

    - Clean finish surfaces and partition accessories.

  - TESTING

    - Provide test report, certified by an independent testing authority, that installed products meet or exceed specified performance.  Provide testing report at no extra cost to Owner.

END OF SECTION