PART 1 - GENERAL

1.01	SUMMARY

A.	Provisions of Division 01 apply to this section.

B.	Section Includes: Complete book theft detection system with alarm, detection targets/strips, and target detuning/deactivators as indicated on Drawings.

C.	Related Sections:

1.	Division 16:  Electrical.

1.02	SYSTEM DESCRIPTION

A.	Performance Requirements:

1.	Library book detection system, by means of exit antennas and radio signals, detects library materials marked with pressure sensitive detection targets/strips. When items pass between the installed antenna system and have not been checked out correctly, the system emits an alarm signal notifying library personnel of an unauthorized removal of items.

2.	An exit aisle shall be formed by a minimum of 2 sensing antennas, with exiting between the antennas.  Aisles shall be not greater than 6-feet wide.

B.	Installation Requirements:

1.	Sensing antennas and peripherals shall be bolted directly to concrete floor with cement lag bolts and anchors.

2.	Power is provided to antennas by installer through conduit installed in floor as part of the Work of Division 16.

1.03	SUBMITTALS

A.	Shop Drawings:  Indicate installation and interface with related Work, including details of assembly and wiring diagrams.

B.	Product Data:  Manufacturer's specifications and other data needed to provide compliance with specified requirements.

C.	Samples:  Detection Target Strips.

1.04	QUALITY ASSURANCE

A.	Qualifications of Installer: Work of this section shall be performed by installers trained by the equipment manufacturer.

B.	Regulatory Requirements:

1.	Entire system, including gates, shall be designed to be in compliance with ADA, CBC and ANSI accessibility requirements and recommendations.

2.	Systems shall be UL listed.

1.05	DELIVERY, STORAGE AND HANDLING

A.	Deliver specified components to the Project site in unopened cartons, crates or suitable packaging bearing manufacturer's label.

B.	Store components in a location safe from weather and damage by construction operations.

1.06	CLOSEOUT REQUIREMENTS

- Staff Instruction: Before Substantial Completion, provide Owner personnel a 4 hour instruction period. Representative of book detection system manufacturer shall instruct library staff members in operation and installation of detection targets/strips in library materials.  Coordinate and arrange the time of staff instruction.

PART 2 - PRODUCTS

2.01	MANUFACTURERS

A.	Liberty System, by Checkpoint System Inc., or equal.

2.02	BOOK DETECTION SYSTEM, GENERAL

A.	Liberty PX Detection System Series as the basis of design.

1.	Lattice dimensions:  68 inches high by 19-3/4 inches wide by 1-1/2 inches deep.

2.	Impact resistant, molded through color thermoplastic.

3.	Weight:  25 pounds.

4.	Floor-mounted with at least 2 cement lag bolts per antenna, minimum 3/8 inch diameter.

5.	Maximum coverage between antennas:  72 inches (center to center).

B.	Operator Control Module:

1.	Electric Power Requirement:  Electronics in system shall be solid-state and operate from a standard 120 volt, 50/60 Hz AC 3-wire grounded outlet.

2.	Location Requirement: Must be within 25 feet of nearest sensing antenna.

3.	Specifications:

a.	Housing:  6 inches long by 4 inches wide by 3-1/2 inches high.

b.	Weight:  2 pounds

c.	Key-controlled on/off switch.

d.	Power-on indicator light (DC).

4.	Theater Rope:

- Multifilament Polypropylene Rope, Solid Braided, 5/8 inches diameter.

- Provide one theater rope to close open space between exterior side of lattice and walls and circulation desk.

2.03	DETECTION TARGET/STRIPS

A.	Radio Frequency Targets:

1.	Series 320 target, 2 inches by 2 inches, with adhesive coating.

2.	Quantity:  Provide 2,000 RF targets and 12,000 RF Detuning Check Cards – White Grid.

3.	Flexible programmed tag laminated on one side with paper stock pressure-sensitive adhesive coating or reversed side with siliconized release paper.

4.	Installed on/in library materials:

a.	Inside front or back covers.

b.	On any page.

c.	Under book jacket.

d.	Printed bookplate.

e.	Under bookplate.

f.	Inside book pocket.

g.	Under book pocket.

h.	Inside or in audio-visual materials or their containers or jackets.

PART 3 - EXECUTION

3.01	EXAMINATION AND PREPARATION

A.	Examine areas and conditions under which Work will be performed to determine best location for sensing unit, and to insure prerequisite Work has been installed, including:

1.	Electrical convenience outlet (120 VAC, 50/60 Hz, AC 3-wire with ground) for connecting system power supply box to be mounted on wall adjacent to outlet.

2.	Conduit (3/4-inch) and pull-wire in floor from power-supply box to antenna locations.  Stub up in first antenna, then stub down and up to second antenna.

3.	Electrical convenience outlet (120 VAC, 50/60 Hz, AC 3-wire with ground) for operator control module to be installed within 25 feet of nearest radio frequency sensing antenna, usually located behind circulation desk.

B.	Assure no metal is installed above the floor within 18 inches of antenna locations.

C.	Coordinate with related Work for drilling proper holes through charging desk for access to power outlet at circulation desk for hook-up of sensing unit.  Holes shall be concealed and shall not interfere with functioning of desk storage spaces and other equipment to be installed in desk.

3.03	DETECTION SYSTEM INSTALLATION

A.	Buried Cable Mounted Components:  Install antennas on top of final finish flooring and anchor to concrete base flooring.

B.	Install wiring and other components, make final connections and adjustments.

C.	Provide all means necessary to eliminate or identify electrical and magnetic interference affecting operation of detection system.

D.	Connect electronic devices to power source and place system in operating condition.

  - OPERATOR CONTROL MODULE INSTALLATION

- Control Module:  Install module within circulation desk as indicated on Drawings and/or as required for operation at the Circulation Desk.

- Provide all means necessary to eliminate or identify electrical and magnetic interference affecting operation of detection system.

- Connect electronic devices to power source and place system in operating condition.

3.06	CLEANUP

A.	Remove rubbish, debris and waste materials and legally dispose of off the Project site.

3.07	PROTECTION

A.	Protect the Work of this section until Substantial Completion.

END OF SECTION