- - GENERAL

  - SECTION INCLUDES

    - Pre-construction subterranean termite treatment below building areas.

  - SUBMITTALS

    - Product data describing materials, product criteria and product manufacturer's recommended instructions for application.

    - Written certification of Applicator's California State License to Practice Insect Extermination.

  - QUALITY ASSURANCE

    - Manufacturer:  Company specializing in manufacture of specified insecticide materials with 5 years minimum experience.

    - Applicator:  Company specializing in pre-construction termite control with 5 years minimum experience.

    - Pre-installation Conference

      - Convene two weeks before starting Work of this Section.

  - GUARANTEE

    - Provide 5 year unlimited guarantee to maintain premises termite free at no additional cost to Owner.

  - SEQUENCING/SCHEDULING

    - Coordinate and schedule Work under this Section to accommodate earthwork and concrete pour operations.

- - PRODUCTS

  - INSECTICIDE

    - Provide EPA-registered termiticide complying with requirements of authorities having jurisdiction, in soluble or emulsible, concentrated formulation that dilutes with water or foaming agent, and formulated to prevent termite infestation.  Use only soil treatment solutions that are not harmful to plants.  Provide quantity required for application at the label volume and rate for maximum termiticide concentration allowed for each specific use, according to product's EPA-Registered Label.

    - Acceptable Products:

      - Demon TC, manufactured by Syngenta Corp.

        - Ingredients:  Cypermethrin, 25.3 percent, inert materials 74.7 percent by weight.

        - Weight:  2 pounds Cypermethrin per gallon.

      - Prevail Termiticide, manufactured by FMC Corporation, Agricultural Products Group, Philadelphia, PA.

        - Ingredients:  Cypermethrin, 24.8 percent, inert materials 75.2 percent by weight.

        - Weight:  2 pounds Cypermethrin per gallon.

      - Or equal as approved in accordance with Division 1, General Requirements for substitutions

- - EXECUTION

  - EXAMINATION

    - Verify that conditions are dry and ready to receive work.

    - Beginning of application means applicator accepts existing conditions.

  - PREPARATION

    - General:  Comply with most stringent requirements of authorities having jurisdiction and with manufacturer's written instructions for preparing substrate.  Remove extraneous sources of wood cellulose and other edible materials such as wood debris, tree stumps and roots, stakes, formwork, and construction waste wood from soil and around foundations.

    - Soil Treatment Preparation:  Remove foreign matter and impermeable soil materials that could decrease treatment effectiveness on areas to be treated.  Loosen, rake, and level soil to be treated, except previously compacted areas under slabs and footings.  Termiticides may be applied before placing compacted fill under slabs if recommended by termiticide manufacturer.

    - Fit filling hose connected to water source at site with backflow preventer, complying with requirements of authorities having jurisdiction.

  - APPLYING SOIL TREATMENT

    - Application:  Mix soil treatment termiticide solution to uniform consistency.  Provide quantity required for application at label volume and rate for maximum specified concentration of termiticide, according to manufacturer's EPA-Registered Label, to following so that continuous horizontal and vertical termiticidal barrier or treated zone is established around and under building construction.  Distribute treatment evenly.

      - Slabs-on-Grade and Basement Slabs:  Under ground-supported slab construction, including footings, building slabs, and attached slabs as overall treatment.  Treat soil materials before concrete footings and slabs are placed.

      - Foundations:  Adjacent soil including soil along entire inside perimeter of foundation walls, along both sides of interior partition walls, around plumbing pipes and electric conduit penetrating slab, and around interior column footers, piers, and chimney bases; and along entire outside perimeter, from grade to bottom of footing.  Avoid soil washout around footings.

      - Masonry:  Treat voids.

      - Penetrations:  At expansion joints, control joints, and areas where slabs will be penetrated.

    - Avoid disturbance of treated soil after application.  Keep off treated areas until completely dry.

    - Protect termiticide solution, dispersed in treated soils and fills, from being diluted until ground-supported slabs are installed. Use waterproof barrier according to EPA-Registered Label instructions.

    - Post warning signs in areas of application.

    - Reapply soil treatment solution to areas disturbed by subsequent excavation, grading, landscaping, or other construction activities following application.

  - WET CONDITIONS

    - Do not apply treatment to wet areas.

    - When rain occurs after treatment, application shall be repeated when soils have dried.

END OF SECTION