- - GENERAL

  - SECTION INCLUDES

    - Parking-stall line and curb painting.

    - Traffic symbols, directional arrows, lettering and safety zones, loading zone.

    - Accessibility signage.

    - Fire lanes.

  - REFERENCES

    - SSPWC - Standard Specifications for Public Works Construction, 2016 Edition.

    - AQMD - Air Quality Management District.

    - Fed. Std 595B - Colors Listed in Government Procurement.

    - 2019 CBC.

  - SUBMITTALS

    - Product data.

    - Shop drawing layout of complete parking lot, indicating stalls, lettering, safety zones, directional arrows, widths of lines and colors.

  - QUALITY ASSURANCE

    - Product Manufacturer:  Company specializing in manufacturing quality traffic line paint products with ten years experience.

    - Applicator:  Company specializing in commercial pavement painting with five years experience.

    - Regulatory Requirements

      - Conform to Federal Regulations concerning lead content of paints.

      - Conform to AQMD, Local Regulations. Copy of regulation is on file at Architect's office.

    - Field Samples

      - Provide field sample in form of one parking lot stall, illustrating coating color, width of stroke, thickness of application and dimensioning.

      - Locate where approved.

      - Accepted sample may remain as part of Work.

      - Do not proceed with pavement marking until sample panel has been approved.

  - DELIVERY, STORAGE AND HANDLING

    - Deliver products to site in sealed and labeled containers.

    - Container labeling to include manufacturer's name, type of paint, brand name, brand code, coverage, surface preparation, drying time, cleanup, color designation and instructions for mixing and reducing.

    - Store paint materials at minimum ambient temperature of 45 degrees F and a maximum of 90 degrees F, unless otherwise recommended by manufacturer.

- - PRODUCTS

  - MANUFACTURERS

    - Products of following manufacturers form basis for design and quality intended.

      - Vista Paint Corporation, Fullerton, CA.

      - Dunn-Edwards Corporation, Los Angeles, CA.

    - Or equal as approved in accordance with Division 1, General Requirements for Substitutions.

  - MATERIALS

    - Traffic Line Paint:

      - Vista Paint: On-Line Traffic Paint.

      - Dunn-Edwards:  VIN-L-STRIPE TRAFFIC PAINT, VINYL EPOXY EMULSION, W801.

    - Striping, pavement markings, and curb markings in accordance with Sections 210-1 and 310-5.6 SSPWC.

  - COLORS

    - As per Drawings

    - Accessible Stalls and Signage:  Blue, conforming to No. 15090 Fed. Std 595B.

    - Parking stalls, lettering, arrows, and traffic signage:  White.

- - EXECUTION

  - INSPECTION

    - Verify that surfaces are ready to receive Work as instructed by product manufacture.

  - APPLICATION

    - Surfaces to be painted shall be clean and free of dust, dirt, grease, oil, water or other contaminates.

      - Existing lines to be removed shall be sandblasted clean.

    - Traffic paint shall not be applied until seal coat has been in place minimum of 10 days.

    - Apply material by machine spray, airless sprayer, roller or brush to provide a minimum thickness of 15 mils average.  Precise edges required, no overspray allowed.

    - Perform Work in accordance with approved Shop Drawings.  Conform to Section 310-5.6.8, SSPWC and CAS/CAR.

    - Painted lines and markings on pavement shall be 4 inches minimum wide and color per Drawings.

  - DEFECTIVE WORK

    - Remove any paint that demonstrates evidence of checking, cracking, peeling, discoloration, lack of bonding or poor coverage.  Misplaced lines shall be completely removed by paint remover or wet sandblasting per Section 310.5.6.3, SSPWC. Painting over misplaced lines will not be permitted.

END OF SECTION