PART 1 - GENERAL

  - RELATED DOCUMENTS

- Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

  - SUMMARY

A.	Section Includes

- Required coordination by General Contractor for Synthetic Running Track Surfacing to be furnished by others under separate contract.

- The work under this section shall consist of General Contractor furnishing all labor and equipment necessary to install, in place, all synthetic running track surfacing and other materials as indicated on the plans and as specified herein. The installation of all new materials shall be performed in strict accordance with these specifications, the track surfacing provider’s instructions and in accordance with all details and shop drawings.

- Work by Others: Synthetic Track Surfacing and Materials to be provided under a CMAS contract.

  - Synthetic Track Surfacing shall be Rekortan® RT - Running Track Resurfacing: Full pour Rekortan® polyurethane, fluid applied, impermeable track resurfacing with an embedded EPDM rubber finish supplied by Advanced Polymer Technology.

  - Synthetic Track manufacturer shall provide to General Contractor adequate information for GC to perform its portion of the Work.

# REFERENCES

    - ASBA - American Sports Builders Association

    - ADA - Americans with Disabilities Act of 1990, as amended

    - ADA Standards – ADA Title II regulations and the 1991 DOJ Standards for Accessible Design

    - CBC - California Building Code

    - CBC 10 – CBC Chapter 10, Means of Egress

    - CBC 11B - CBC Chapter 11B, Access to Public Buildings, Public Accommodations, Commercial Facilities and Publicly Funded Housing

    - CIF Standards

    - IAAF - International Amateur Athletic Federation

    - NCAA - National Collegiate Athletic Association

    - NFHS - National Federation of State High School Associations

    - USTC – United States Tennis Court TBA –Track Builders Association

    - TBA Manual - USTC/TBA Track Construction Manual

# ACTION SUBMITTALS

    - Shop Drawings showing layout of track striping and markings

    - Statement of Qualifications from installer

    - IAAF / NCAA Certificates of Marking Accuracy

    - Manufacturer’s Maintenance Instructions

# QUALITY ASSURANCE

    - The track surface installer shall be authorized by Manufacturer and possess a minimum of five (5) years’ experience installing the Rekortan® systems.

    - The supervisor, of the installing company, must have five (5) years’ experience surfacing Rekortan® systems and must have installed a minimum of ten (10) World Athletics approved track systems within the last three (3) years.

    - The manufacturer must offer a minimum of four (4) World Athletics approved track systems.

    - Submit evidence that the synthetic surfacing contractor is a member of the American Sports Builders Association (ASBA).

# DELIVERY, STORAGE, AND HANDLING

    - Deliver materials in their manufacturer’s original, unopened, undamaged packing with labels intact and legible.

      - Inspect materials promptly upon delivery for defects, missing, and non-complying items. Make claims, or place orders to obtain proper materials in a timely manner; do not delay the orderly progress of the work.

    - Storage and handling of materials shall be in accordance with manufacturer’s recommendations.

# SITE CONDITIONS

    - Weather: Surfacing shall not be done when the threat of freezing exists for the following 24 hours, rain is imminent or gusting winds are occurring.

    - During any surfacing and striping, sprinkler systems must be shut off or controlled so that no water falls on the track or event area surfaces. Other trades and school district personnel must stay off the wet or curing surface.

    - Do not apply rubberized topping when base surface temperature is less than 50° F.

    - Only mix and apply when meeting manufactures recommended guidelines.

    - The General Contractor shall provide temporary barriers as required to prevent public entry to construction area and to protect adjacent properties from damage during construction operation.

# WARRANTY

    - Manufacturer and installer shall jointly warrant installed track surfacing to be and to remain free from defects in material and installation for a period of at least 5 years from the Date of Substantial Completion. Upon written notice from Owner, they shall promptly, without cost, and with least practicable inconvenience to Owner correct such defects.

      - Evidence of defect in material, installation or shall be one or more of the following individually or in any combination:

- Excessive color change or fading

- Excessive wear

- Delamination between layers or between track and substrate

      - Upon expiration of the Maintenance Service Contract, Owner will maintain track in accordance with manufacturer’s instructions.

# MAINTENANCE

    - Provide a 2-year maintenance service contract including the following.

      - Conduct manufacturer’s recommended maintenance procedures, except routine sweeping, during the maintenance contract period.

      - Provide reconditioning and restriping of the track within 45 days of expiration of maintenance contract; scheduled at District’s convenience.

    - Immediately prior to expiration of the maintenance contract:

      - Furnish to the District one of each tool and special piece of equipment, minimum 5- gallons of cleaning solution, and other materials required to conduct the manufacturer’s recommended maintenance procedures.

      - Instruct District personnel	in manufacturer’s	recommended maintenance procedures.

# PART 2 - PRODUCTS

        - ACCESSORIES

          - Marking Paint: aliphatic urethane, traffic marking paint compatible with track surfacing system.

# PART 3 - EXECUTION

  - EXAMINATION

    - Verify existing track surface for dimensional stability, strength, and surface integrity. Notify owner of any deficiencies.

    - Do not begin installation until unsatisfactory conditions are corrected. Beginning installation means acceptance of existing conditions including preparatory work of others, if any.

    - Entire surface shall be pressure washed.

# PRODUCT AND MATERIAL DESCRIPTION

    - Rekortan® RT with encapsulation is a 5 mm, impermeable full pour system. It consists of a fluid- applied topcoat of Rekortan® polyurethane and embedded EPDM rubber granules encapsulated with a single component, aliphatic waterborne coating.

    - Primer: Rekortan® primer designed to go over existing polyurethane surfaces shall be used.

    - Full Pour Polyurethane: The full pour polyurethane shall be made of a high quality, two- component Rekortan® polyurethane.

    - Colored Rubber (EPDM). SBR rubber is not allowed in the wearing course.

    - Single Component Waterborne, aliphatic, encapsulation coating. Coating cannot contain isocyanates or solvents.

# INSTALLATION

    - The entire track surface shall be pressure washed prior to the application of the primer.

    - Prime entire existing track surface with a compatible Rekortan® polyurethane primer. Primer shall dry to a tack-free condition prior to beginning next application. Do not allow primed surface to become contaminated by rain, dust, debris, etc.

    - Resurfacing Layer: After the primer has cured to tack free, mix the Rekortan® full pour coating and apply using a notched trowel or squeegee and allow to self-level. Broadcast to excess with colored EPDM granules, using a flat shovel or machine spreader ensuring the coating is covered.

    - Allow EPDM layer to cure and reclaim all excess rubber by means of a sweeper and/or vacuum/blower without causing damage to the track surface.

    - Install two coats of a single component waterborne coating.

# STRIPING

    - All line marking paint shall be compatible and approved for the synthetic surfacing. Only an experienced track striping specialist shall perform the line striping.

    - Lane Striping and Markings. Apply line paint to meet rules and regulations of NFHS.

    - Contractor is to re-stripe existing 8-lane track surfacing including numbers, markings, letters, etc. as part of this project.

# PROTECTION

    - Protect completed surfaces from	damage during installation and subsequent construction activities until Date of Substantial Completion.

END OF SECTION