PART 1 - GENERAL

1.01	WORK INCLUDED

- Floor Sinks

- Water Closets

- Lavatories

- Service Sink

- Sinks

- Fixture Trim

- Urinals

- Toilet Seats

- Shower Units

1.02	RELATED WORK

- Section 22 05 29…	Supports and Anchors

- Section 22 10 00…	Plumbing Piping

- Section 22 16 00….	Plumbing Specialties

1.03	REFERENCES

- ANSI A112.18.1—Finished and Rough Brass Plumbing Fixture Fittings

- ANSI A112.19.2—Vitreous China Plumbing Fixtures

- ANSI A112.19.5—Trim for Water-Closet Bowls, Tanks, and Urinals

- ANSI A 117.1    – 	Accessible and Usable Buildings and Fixtures

1.04	QUALITY ASSURANCE

A.	Fixtures:  By same manufacturer for each product specified throughout, except as indicated otherwise.

B.	Trim:  By same manufacturer for each product specified throughout, or as stated.

1.05	SUBMITTALS

A.	Submit product data under provisions of Section ____, General Conditions, and Supplementary General Conditions.

B.	Include fixtures, sizes, rough-in dimensions, utility sizes, trim, and finishes.

1.06	OPERATION AND MAINTENANCE DATA

A.	Submit operation and maintenance data.

B.	Include fixture trim exploded view and replacement parts lists.

1.07	WARRANTY

A.	Provide one year manufacturer's warranty.

PART 2 - PRODUCTS

2.01	GENERAL

- Refer to Plumbing Fixture Schedule on drawings for further information.

2.02 	WATER CLOSETS AND URINALS

- Kohler and American Standard fixtures

- “Waterless Urinals are PROHIBITED

- Flushometers shall be Sloan “Royal” Series

- Water Closet Seats shall have Stainless Steel (S/S) Post and Self-sustaining Hinges

- PVC Closet Flanges shall be Stainless Steel (S/S) Bolt Ring

- Offset closet flanges are prohibited

2.03 	LAVATORIES

- Fixtures: Kohler or American Standard

- Faucets: Zurn

- Wall Hung Lavatories: Vitreous China

2.04 	SINKS

All Countertop Mounted Sinks shall be 18 Gauge full undercoated Stainless Steel (S/S)

PART 3 – EXECUTION

3.01	INSPECTION

A.	Review millwork shop drawings.  Confirm location and size of fixtures and openings before rough-in and installation.

B.	Verify adjacent construction is ready to receive rough-in work of this Section.

3.02	INSTALLATION

A.	Provide 17gauge polished chrome trap at each fixture.

B.	Provide chrome plated stops with rigid supplies to fixtures with loose key stops, reducers, and escutcheons.  All stops shall have renewable seats and disks.

C.	Install components level and plumb.

D.	Install and secure fixtures in place with wall carriers and bolts.

E.	Seal fixtures to wall and floor surfaces with Silicone sealant as to match fixture.

F.	Install all handicapped fixtures per ANSI A117.1.

G.	All Flushometers Tailpieces shall be installed with Stand-Off Brackets.

H.	Water Closet Flanges must be Permanently Attached.

I.	Assemble all fixtures and equipment shipped loose which this Contractor is required to make final connections to, whether furnished by him or by others (i.e., science cabinet water faucets, trim, or tailpiece, etc.).

J.	Provide a pre-manufactured drain insulating cover on all sinks and lavatories designated as a handicap fixture.  Refer to plumbing drawings and the Architect’s drawings for designation.

3.03	ADJUSTING AND CLEANING

A.	Adjust stops or valves for intended water flow rate to fixtures without splashing, noise, or overflow.

B.	At completion clean plumbing fixtures and equipment.

3.04	FIXTURE ROUGH-IN SCHEDULE

Rough-in fixture piping connections in accordance with table of minimum sizes for particular fixtures shown with fixture schedule.  (Refer to drawings.)

END OF SECTION